ESP32-C6 SuperMini

Smart Relay PCB

Complete PCB Design Guide

WiFi 6 • Bluetooth 5 • Zigbee • Thread • Matter HF3FF Relay • HLK-PM01 PSU • PC817 Isolation • Energy Monitoring

Commercial Smart Home / Industrial IoT Platform

| Feature | Specification | Notes |
| --- | --- | --- |
| MCU | ESP32-C6 SuperMini | Wi-Fi 6, BLE5, Zigbee, Thread |
| Relays | 1 to 4 (expandable) | HF3FF SPDT 10A 250VAC |
| Power Supply | HLK-PM01 | 230VAC → 5V / 600mA |
| Isolation | PC817 / EL817 | MCU ↔ Relay coil |
| Energy Meter | HLW8012 or BL0937 | V, I, P — 1% accuracy |
| Fuse | Blade/glass fuse holder | 5A or 10A rated |
| Protocols | ESPHome / Tasmota | Matter/Thread ready |

# 1. Project Overview & Architecture

This guide covers the complete design and build of a compact, commercial-grade smart relay PCB based on the ESP32-C6 SuperMini. The board is designed to control mains-voltage loads (lights, fans, appliances) from ESPHome or Tasmota firmware with support for future Thread/Zigbee mesh networking.

## 1.1 System Block Diagram

Power Path (Mains Side)

230VAC Mains Input → Blade Fuse (10A) → HLK-PM01 AC-DC Converter → 5V DC Rail

5V DC → 3.3V LDO (AMS1117-3.3) → ESP32-C6 SuperMini + PC817 + HLW8012

5V DC also powers Relay Coil (via transistor driver — isolated from MCU)

Current Shunt in Live wire → HLW8012 or BL0937 for energy measurement

Control Path (Low-Voltage Side)

ESP32-C6 GPIO → 1kΩ resistor → PC817 LED (anode) → PC817 collector → NPN Base (S8050)

NPN Collector → Relay Coil+ → 5V rail

Flyback Diode (1N4007) across Relay Coil to suppress back-EMF spike

Each relay channel: 1× PC817 + 1× S8050 NPN + 1× 1N4007 + 1× HF3FF relay

Load Path (Mains Switching)

Live wire → Fuse → HLW8012 Shunt → Relay COM pin

Relay NO pin → Terminal block → Load (lamp/appliance)

Neutral wire → Direct to terminal block (not switched)

Creepage/clearance: minimum 3mm for 250VAC on PCB traces

## 1.2 Component Selection Summary

| Component | Part Number | Key Specs |
| --- | --- | --- |
| MCU Module | ESP32-C6 SuperMini | Wi-Fi6, BLE5, Zigbee3, Thread, 3.3V, 160MHz |
| Power Supply | Hi-Link HLK-PM01 | 85-264VAC → 5V/600mA, isolated, CE/UL |
| Relay (per ch.) | Hongfa HF3FF/005-1ZST | 5V coil, 10A SPDT, 250VAC, 19×15×15mm |
| Optocoupler | PC817 or EL817 | CTR 80-160%, 50mA, DIP-4, drop-in compatible |
| NPN Driver | S8050 or 2N2222 | TO-92, Ic=1.5A, Vceo=40V, hFE≥100 |
| Flyback Diode | 1N4007 | 1A/1000V, across each relay coil |
| Energy IC | HLW8012 or BL0937 | Pulse output, ESPHome native, 1% accuracy |
| Shunt Resistor | 1mΩ (or 2×2mΩ parallel) | 2W rated, mains side current sense |
| LDO Regulator | AMS1117-3.3 SOT-223 | 5V→3.3V, 1A, for MCU supply |
| Fuse Holder | 5×20mm PCB holder | 10A glass fuse, mains series protection |
| Terminal Blocks | KF300 / KF350 2.54mm | AC Line, Neutral, Load outputs |
| Filter Cap | 100µF/16V electrolytic | On 5V rail after HLK-PM01 |

⚠️ SAFETY NOTE: This board handles 230VAC mains voltage. Always design with adequate creepage (3mm min between HV and LV) and clearance. Enclose in a suitable IP-rated box. Follow IEC 60335 guidelines for appliance control.

# 2. Schematic Design — Step by Step

## 2.1 Power Supply Sub-Circuit (HLK-PM01)

The HLK-PM01 is a fully isolated AC-DC converter module in a compact SIP package. It handles the mains isolation internally, making your design simpler and certifiable.

### HLK-PM01 Connections:

| Pin / Net | Connection |
| --- | --- |
| AC/L (Pin 1) | Mains LIVE wire (after fuse) |
| AC/N (Pin 2) | Mains NEUTRAL wire |
| +Vo (Pin 3) | 5V DC output → 100µF cap to GND, then to 5V rail |
| -Vo (Pin 4) | GND (DC common) |
| 5V Rail | → AMS1117-3.3 input → relay coils → PC817 |
| 3.3V Rail | AMS1117 output → ESP32-C6, HLW8012, pull-ups |

### AMS1117-3.3 LDO Circuit:

| Component | Value / Notes |
| --- | --- |
| C_in (input cap) | 10µF/10V ceramic, close to AMS1117 input pin |
| C_out (output cap) | 10µF/10V + 100nF ceramic in parallel, output pin |
| Bulk cap on 5V | 100µF/16V electrolytic + 100nF ceramic |
| Bulk cap on 3.3V | 47µF/10V electrolytic + 100nF ceramic |

💡 TIP: The HLK-PM01 can supply 600mA. Budget: AMS1117 quiescent + ESP32-C6 peak (~200mA WiFi) + HLW8012 (~5mA) + PC817 inputs (~5mA each) + relay coils (~72mA each at 5V). For 4 relays: ~200 + 20 + 4×72 = ~508mA. Stay within limit. For 4+ relays, consider HLK-PM03 (3W/600mA@5V) or HLK-5M05 (5W/1A@5V).

## 2.2 Relay Driver Circuit (Per Channel)

Each relay channel uses a PC817 optocoupler for isolation between the 3.3V MCU side and the 5V relay coil side, followed by an S8050 NPN transistor to provide enough current to drive the relay coil.

| Component | Value & Function |
| --- | --- |
| R_gpio (R1) | 1kΩ — current limit for PC817 LED (GPIO → PC817 pin 1) |
| PC817 (U_opt) | Optocoupler — isolates 3.3V MCU from 5V relay side |
| R_pull (R2) | 10kΩ — pull-up from 5V to PC817 collector (pin 4) |
| R_base (R3) | 1kΩ — base resistor for S8050 NPN transistor |
| S8050 (Q1) | NPN TO-92 — collector to relay coil+, emitter to GND |
| D_flyback (D1) | 1N4007 — anode to GND, cathode to relay coil+ (anti back-EMF) |
| HF3FF Relay | Pin 1/2 = coil (pin1 to collector of S8050, pin2 to 5V) |
| Terminal Block | Relay COM → fuse/shunt output; Relay NO → load terminal |

### Signal Flow (GPIO LOW = Relay ON, active-low logic):

Relay Driver Logic

GPIO = LOW → PC817 LED ON → PC817 phototransistor conducts → base of S8050 driven → S8050 saturates → Relay coil energized → NO contact closes

GPIO = HIGH (or floating with pull-up) → PC817 LED OFF → S8050 OFF → Relay de-energized → NO contact open

Flyback diode D1 conducts during de-energization to absorb coil spike

In ESPHome: set 'inverted: true' on GPIO output pin if using active-low

💡 COIL VOLTAGE: Use HF3FF/005-1ZST (5V coil version) to match the 5V rail directly. The 5V coil resistance is ~70Ω, coil current = ~71mA per relay. The S8050 can handle up to 500mA, well within spec.

## 2.3 Energy Monitoring Sub-Circuit (HLW8012 / BL0937)

Both HLW8012 and BL0937 are drop-in compatible and use the same ESPHome driver. Place a current shunt (low-value resistor) in series with the LIVE wire on the mains side. The chip outputs frequency-modulated pulses for voltage, current, and power.

| Pin | Connection |
| --- | --- |
| VCC (Pin 5) | 5V from 5V rail (chip requires 5V, not 3.3V) |
| GND (Pin 8) | GND |
| CF (Pin 6) | Power pulse output → GPIO (e.g., GPIO20) via 10kΩ pull-up to 3.3V |
| CF1 (Pin 7) | Voltage/Current pulse → GPIO (e.g., GPIO21) via 10kΩ pull-up to 3.3V |
| SEL (Pin 1) | GPIO (e.g., GPIO22) — HIGH=Current mode, LOW=Voltage mode |
| IP+ / IP- (Pins 2/3) | Current shunt inputs — 1mΩ shunt in Live wire (mains side) |
| VP (Pin 4) | Voltage divider input — R divider from Live (mains) to chip |

### Voltage Sensing Divider (Mains → HLW8012 VP pin):

| Resistor | Value & Notes |
| --- | --- |
| R_v1 (high-side) | 470kΩ + 470kΩ in series = 940kΩ (split for safety, 2× 470kΩ 1/4W) |
| R_v2 (low-side) | 1kΩ (to GND) — check chip datasheet for exact ratio |
| C_vp | 10nF ceramic cap from VP pin to GND — noise filter |
| Current Shunt | 1mΩ / 2W resistor. Or use 2× 2mΩ 1W in parallel |
| C_ip | 10nF across IP+/IP- — EMI filtering |

⚠️ ISOLATION WARNING: The HLW8012/BL0937 VP and IP pins are connected to mains voltage through resistors. These are NOT isolated. Keep a minimum 5mm spacing between any HLW8012 mains-connected pad and the ESP32-C6 / LV components. Do NOT share GND between mains sense path and digital GND without proper design.

## 2.4 Fuse Protection

| Item | Specification |
| --- | --- |
| Fuse Type | 5×20mm glass fuse or automotive blade fuse |
| Fuse Rating | 10A for 10A relay board; use 5A if loads are lighter |
| Fuse Holder | Keystone 3557 or similar PCB-mount 5×20mm holder |
| Placement | Series with Mains LIVE input, before HLK-PM01 and relay contacts |
| Marking | Label F1 on silkscreen. Add 'FUSE 10A 250VAC' text on PCB |
| TVS / MOV | Optional: 275VAC MOV (Varistor) across AC input for surge protection |

# 3. PCB Layout — Step by Step in KiCad

## 3.1 Board Setup & Design Rules

Before placing any component, configure your board settings to meet safety requirements for mains-voltage PCB design.

| Parameter | Value |
| --- | --- |
| Board Layers | 2-layer (F.Cu + B.Cu) |
| Copper Weight | 2 oz (70µm) — required for mains traces carrying 10A |
| PCB Thickness | 1.6mm FR4 |
| Min Track Width (LV) | 0.25mm for signal; 0.5mm for 3.3V/5V power |
| Min Track Width (Mains) | 2.0mm minimum (10A @ 2oz copper, 10°C rise) |
| Creepage (HV to LV) | 3.0mm minimum (for 250VAC working voltage, IEC60065) |
| Clearance (HV to LV) | 2.0mm minimum |
| Mains copper gap | Set DRC rule: min 3mm clearance between AC_LIVE net and all LV nets |

### KiCad DRC Setup:

- Open PCB Editor → File → Board Setup

- Design Rules → Constraints: set Minimum Clearance = 0.2mm (general)

- Add Net Class 'MAINS': assign AC_LIVE and AC_NEUTRAL nets

- Set MAINS net class clearance = 3.0mm, trace width = 2.0mm

- Add Net Class 'RELAY_LOAD': assign LOAD_L net (after relay contact)

- Set RELAY_LOAD clearance = 3.0mm, trace width = 2.0mm

## 3.2 Component Placement Strategy

The PCB is divided into two strict zones: HIGH VOLTAGE (mains) and LOW VOLTAGE (MCU/logic). Physical separation is mandatory.

Zone A — HIGH VOLTAGE (Right side of board)

HLK-PM01 module (place near AC input terminals)

Fuse holder F1 (on AC LIVE line — first component after input terminal)

HF3FF Relay(s) — stacked horizontally if multiple channels

Relay contact terminals (screw terminals for LOAD wires)

HLW8012/BL0937 + shunt resistor + voltage divider resistors

MOV varistor (optional, across AC input)

AC input screw terminals (L, N)

Slot the PCB between HV and LV zones: 1mm routed slot as air gap reinforcement

Zone B — LOW VOLTAGE (Left side of board)

ESP32-C6 SuperMini module (center-left, antenna pointing away from board edge)

AMS1117-3.3 LDO regulator

PC817 optocouplers (in a row, one per relay channel)

S8050 transistors (next to corresponding PC817)

1N4007 flyback diodes (next to each relay)

Decoupling capacitors for 5V and 3.3V rails

Programming header (2.54mm 4-pin: 3.3V, GND, TX, RX) — optional

Expansion header for additional relay modules

🔑 KEY RULE: No copper trace may cross the HV/LV boundary except: (1) PC817 connections — but keep isolation gap of 3mm between LED side and collector side pads, and (2) power output from HLK-PM01 — routed to LV zone as 5V DC only.

## 3.3 Step-by-Step PCB Layout in KiCad

### Step 1: Import Netlist / Update PCB from Schematic

- In KiCad Schematic Editor: Tools → Update PCB from Schematic

- All components appear as rats-nest in PCB editor

- Do NOT place anything yet — first define board outline

### Step 2: Draw Board Outline

- Switch to Edge.Cuts layer

- For single relay board: draw rectangle 65mm × 45mm

- For 2-relay: 80mm × 45mm; for 4-relay: 120mm × 50mm

- Add mounting holes: 3mm drill at 4 corners (3mm from each edge)

- Use footprint Mounting_Hole_3.2mm_M3 from KiCad standard library

### Step 3: Place HIGH VOLTAGE Components (Zone A — right side)

- Place AC input terminal block at right edge (J_AC_IN): screws facing outward

- Place fuse holder F1 immediately after terminal, in series with LIVE trace

- Place HLK-PM01 next — allow at least 2mm clearance all around its body

- Place HF3FF relay(s) — leave 3mm gap between relay body and any LV components

- Place relay output terminals (J_LOAD) at the right edge or top edge

- Place HLW8012 chip — keep IP+/IP- traces short, connect to shunt

- Place shunt resistor (R_shunt) in series with LIVE wire path (between fuse and relay COM)

### Step 4: Place LOW VOLTAGE Components (Zone B — left side)

- Place ESP32-C6 SuperMini: center-left area, antenna side facing LEFT edge (no copper under antenna)

- Place AMS1117-3.3: near 5V input from HLK-PM01, with caps close

- Place PC817 row: between ESP32 and relays — aligned neatly in a row

- Place S8050 transistors: one next to each PC817, between PC817 and relay coil

- Place 1N4007 diodes: one next to each relay coil connection

- Place all bypass caps: within 2mm of their associated IC's power pin

### Step 5: Route Power Planes

- Create a copper fill/zone for GND on B.Cu (bottom layer) covering the entire LV zone

- Create a copper fill for 5V rail on F.Cu (top layer) in LV zone only — NOT under antenna

- Route mains LIVE and NEUTRAL as 2mm wide traces on F.Cu in HV zone only

- Route 5V output from HLK-PM01 to LV zone as a 1mm trace crossing the zone boundary cleanly

- Add stitching vias (every 5mm) connecting GND zones between top and bottom layers

### Step 6: Route Signal Traces (LV Zone)

- Route GPIO → R1 → PC817 pin1: 0.3mm trace

- Route PC817 pin4 → R_base → S8050 base: 0.3mm trace

- Route S8050 collector → Relay coil pin1: 0.5mm trace (coil current ~72mA)

- Route HLW8012 CF/CF1/SEL to ESP32 GPIOs: 0.3mm traces

- All GPIO traces: keep short, away from relay coil area

### Step 7: Route High-Voltage Mains Traces

- LIVE: input terminal → fuse → shunt → relay COM: 2.0mm trace minimum

- NEUTRAL: input terminal → output neutral terminal: 2.0mm trace

- LOAD: relay NO → load terminal: 2.0mm trace

- Verify 3mm gap between all mains traces and any LV copper

- Add extra solder mask clearance: set zone keepout in HV area

### Step 8: Antenna Keepout Zone

- For ESP32-C6 SuperMini, the PCB antenna is on the USB-C end of the board

- In PCB editor: Place → Rule Area

- Draw rectangle covering 5mm beyond the antenna end of the SuperMini

- Set Rule Area: no copper, no vias, no tracks (all layers)

- Verify no copper pour fills into this zone after fill operation

### Step 9: Silkscreen & Fabrication Markings

- Label all terminal blocks: 'L', 'N', 'GND', 'LOAD1', 'LOAD2' etc.

- Mark fuse rating: 'F1  10A 250VAC'

- Add WARNING text: 'DANGER — 230VAC INSIDE' on top silkscreen

- Mark relay channels: 'CH1', 'CH2' etc.

- Add HV zone marking: dashed outline box around HV area on silkscreen

- Add company/product name, version, date

- Mark antenna keepout on silkscreen as dashed box labeled 'RF KEEPOUT'

### Step 10: Run DRC and Verify

- Inspect → Design Rules Checker → Run DRC

- Fix all errors (unconnected nets, clearance violations, etc.)

- Manually verify: 3mm clearance between AC_LIVE net and any LV net

- Check: no tracks crossing zone boundary except approved paths

- Verify copper pour does not violate antenna keepout

- Print 1:1 PDF and verify component sizes match physical parts

# 4. Expandable Relay Architecture

## 4.1 Base Board (1 or 2 Relays)

The base board hosts the ESP32-C6 SuperMini, power supply, energy monitor, and 1–2 relay channels. It is designed to work standalone or with expansion modules.

| GPIO Assignment | Function |
| --- | --- |
| GPIO0 | Relay CH1 control (active-low via PC817) |
| GPIO1 | Relay CH2 control (active-low via PC817) |
| GPIO2 | Relay CH3 control (via expansion header) |
| GPIO3 | Relay CH4 control (via expansion header) |
| GPIO20 | HLW8012 CF pin (Power pulse) |
| GPIO21 | HLW8012 CF1 pin (V/I pulse) |
| GPIO22 | HLW8012 SEL pin (select V or I mode) |
| GPIO8 | WS2812 RGB LED (onboard SuperMini) |
| GPIO9 | Boot button (also usable as input) |
| GPIO19/20 | USB D-/D+ (reserved for USB programming) |
| GPIO4/5/6/7 | Available for I2C display, extra sensors, etc. |

## 4.2 Expansion Header Design

Add a 2.54mm pitch 8-pin expansion header to the base board. This allows attaching satellite relay boards for additional channels without redesigning the base board.

| Pin | Signal |
| --- | --- |
| 1 | 5V (from main 5V rail — up to 200mA for expansion) |
| 2 | GND |
| 3 | GPIO2 (Relay CH3 control) |
| 4 | GPIO3 (Relay CH4 control) |
| 5 | GPIO4 (Relay CH5 — future) |
| 6 | GPIO5 (Relay CH6 — future) |
| 7 | 3.3V (for logic ICs on expansion) |
| 8 | GND |

## 4.3 Relay Expansion Module (Satellite Board)

Design a simple satellite board with just: relay drivers (PC817 + S8050 + 1N4007 + HF3FF) + screw terminals. The satellite board connects to the base via the 8-pin header. No MCU needed on satellite board.

Satellite Board Contents (per 2 extra relays)

1× 8-pin female header (mates with base board expansion header)

2× PC817 optocouplers

2× S8050 NPN transistors

2× 1N4007 flyback diodes

2× HF3FF/005-1ZST relays (5V coil)

2× 1kΩ resistors (GPIO → PC817)

2× 1kΩ resistors (base resistors)

2× 10kΩ pull-up resistors

Screw terminal blocks for LOAD outputs

Power: directly from 5V and GND on expansion header

💡 POWER NOTE: Each additional relay = ~72mA. Base board HLK-PM01 provides 600mA. After base board consumption (~250mA), ~350mA remains. This safely supports 4 additional relays on satellite boards. For more channels, add a separate HLK-PM01 on the satellite board powered from mains tap.

# 5. Firmware — ESPHome Configuration

## 5.1 ESPHome YAML for Base Board

Below is the complete ESPHome YAML configuration for the base board with 2 relay channels, energy monitoring, and Thread/Zigbee-ready setup.

esphome: name: smart-relay-c6 friendly_name: Smart Relay C6 esp32: board: esp32-c6-devkitc-1 framework: type: esp-idf switch: - platform: gpio pin: number: GPIO0 inverted: true # active-low relay name: Relay 1 restore_mode: RESTORE_DEFAULT_OFF - platform: gpio pin: number: GPIO1 inverted: true name: Relay 2 restore_mode: RESTORE_DEFAULT_OFF sensor: - platform: hlw8012 sel_pin: number: GPIO22 inverted: true cf_pin: GPIO20 cf1_pin: GPIO21 current_resistor: 0.001 # 1mΩ shunt voltage_divider: 940 # 940kΩ / 1kΩ power: name: Active Power unit_of_measurement: W current: name: Current voltage: name: Voltage update_interval: 10s

## 5.2 Thread / Zigbee Future Use

The ESP32-C6 has native Thread and Zigbee support in hardware (IEEE 802.15.4 radio). To switch between WiFi and Thread/Zigbee, only a firmware change is needed — no hardware change required.

| Protocol | How to Enable |
| --- | --- |
| Zigbee | Use ESP-Zigbee-SDK (esp-idf) or ESPHome Zigbee component (experimental). Set board role as Zigbee End Device or Router. |
| Thread | Use OpenThread stack. ESP-IDF provides openthread component. Can run as Thread End Device or Border Router with external network. |
| Matter over Thread | Use esp-matter SDK. Enables Matter-certified device controllable by Apple Home, Google Home, Amazon Alexa. |
| WiFi (current) | ESPHome or Tasmota — fully supported today, native HomeAssistant integration. |

💡 DESIGN NOTE: Since all three radios (WiFi, Zigbee, Thread) share the same 2.4GHz antenna on the ESP32-C6, ensure your antenna keepout zone is maintained. The SuperMini PCB antenna handles all protocols — no external antenna needed for typical room-to-room range.

# 6. Safety, Certification & Commercial Notes

## 6.1 PCB Safety Checklist

| Safety Item | Requirement & Status |
| --- | --- |
| Creepage distance | ≥3mm between HV (AC) traces and LV (DC/signal) — verify in DRC |
| Clearance distance | ≥2mm air gap HV to LV — verify in DRC |
| Mains trace width | ≥2.0mm for 10A on 2oz copper — route and verify |
| Fuse in series | 10A fuse on LIVE wire before any other component — mandatory |
| Isolation barrier | PC817 provides 5000V isolation — do not short across barrier |
| Flyback diodes | 1N4007 across every relay coil — mandatory to prevent MCU damage |
| Thermal rating | HLK-PM01 rated 40°C ambient — ensure adequate ventilation in enclosure |
| PCB material | FR4 minimum — do not use low-grade PCB for mains applications |
| Solder mask | Use on all copper — prevents accidental bridging on HV traces |
| Conformal coating | Recommended for commercial product in humid environments |

## 6.2 India-Specific Regulatory Notes (for Commercial Sale)

| Certification | Notes |
| --- | --- |
| BIS (IS 13252) | Required for electronic products sold in India. Apply via BIS portal. Covers product safety. |
| WPC Approval | Required for devices with wireless (WiFi/BT). ESP32-C6 modules may already have WPC — check your module supplier's docs. |
| EMI/EMC (CISPR 32) | Conducted/radiated emissions test. Required for commercial product. Test at accredited NABL lab. |
| CE / FCC | If exporting to EU or USA. ESP32-C6 modules typically already have these certifications. |
| IP Rating | IP20 minimum for indoor smart switch. IP44 for bathroom/outdoor. Choose enclosure accordingly. |
| BEE Star Rating | If marketed as energy-efficient product, optional but adds market value. |

## 6.3 Enclosure Recommendation

| Parameter | Recommendation |
| --- | --- |
| Material | ABS plastic, UL94-V0 rated (flame retardant) |
| Size (1 relay) | 85mm × 50mm × 35mm DIN rail or wall mount box |
| Size (4 relay) | 120mm × 60mm × 40mm enclosure |
| Ventilation | Small slots on top and sides — HLK-PM01 does generate some heat |
| Cable entry | Rubber grommets or PG7 cable glands for mains wires |
| Terminal access | Consider snap-off section or dedicated access panel for screw terminals |
| Label/marking | Permanent label: Input voltage, output, max load, CE/BIS marks |

# 7. Bill of Materials (BOM) — Single Relay Board

| Reference | Part / Value | Qty × Notes |
| --- | --- | --- |
| U1 | ESP32-C6 SuperMini module | 1× — main MCU |
| U2 | Hi-Link HLK-PM01 | 1× — 230VAC→5V PSU |
| U3 | AMS1117-3.3 SOT-223 | 1× — 5V→3.3V LDO |
| U4 | PC817 or EL817 DIP-4 | 1× per relay — optocoupler |
| U5 | HLW8012 or BL0937 SOP-8 | 1× — energy monitor IC |
| Q1 | S8050 TO-92 NPN | 1× per relay — relay driver |
| RL1 | HF3FF/005-1ZST relay | 1× per relay — 5V coil SPDT 10A |
| D1 | 1N4007 DO-41 | 1× per relay — flyback diode |
| F1 | 5×20mm fuse holder PCB | 1× — 10A 250VAC glass fuse |
| R1 | 1kΩ 0402/0603 | 1× per relay — GPIO current limit |
| R2 | 10kΩ 0402/0603 | 1× per relay — PC817 pull-up |
| R3 | 1kΩ 0402/0603 | 1× per relay — S8050 base resistor |
| R4/R5 | 470kΩ 0.25W (×2) | 2× — mains voltage divider (HLW8012) |
| R6 | 1kΩ 0402 | 1× — HLW8012 VP low-side |
| R_shunt | 1mΩ 2W shunt resistor | 1× — current sensing |
| C1 | 100µF/16V electrolytic | 1× — 5V bulk cap |
| C2 | 47µF/10V electrolytic | 1× — 3.3V bulk cap |
| C3-C6 | 100nF 0402 ceramic | 4× — decoupling caps |
| C7 | 10µF/10V ceramic | 2× — AMS1117 in/out |
| J1 | KF350 2P screw terminal | 1× — AC input (L, N) |
| J2 | KF350 3P screw terminal | 1× — LOAD output (COM, NO, NC or Load+N) |
| J3 | 2.54mm 4-pin header | 1× — programming header (opt) |
| J4 | 2.54mm 8-pin header | 1× — expansion header |
| RV1 | 275VAC MOV 10mm | 1× — optional surge protector |
| PCB | FR4 2-layer 2oz 1.6mm | 1× — custom board from JLCPCB/PCBWay |

💰 COST ESTIMATE (India): Components per unit ~₹250–350 (single relay). PCB from JLCPCB ~₹80–120 for 5 pieces. Enclosure ~₹80–150. Total per unit at prototype qty: ~₹400–600. At volume (100+): ~₹250–350 per unit. Target retail price: ₹999–1499 for single relay smart switch.

# 8. Production Workflow — Prototype to Production

## 8.1 Prototype Phase

- Complete schematic in KiCad → verify with simulation or breadboard test

- Complete PCB layout → run DRC → fix all errors → generate Gerbers

- Order 5 PCBs from JLCPCB or PCBWay (2-layer, 2oz copper, ENIG finish recommended)

- Source components from LCSC or Mouser — buy 10× for first build

- Assemble manually: start with SMD components, then through-hole, then connectors

- Power-on test: measure 5V and 3.3V rails BEFORE connecting ESP32

- Flash ESPHome firmware via USB-C

- Test relay switching: verify NO/NC contacts with multimeter

- Test energy monitor: compare HLW8012 readings with calibrated meter

- Test WiFi connectivity, HomeAssistant integration

## 8.2 PCB Assembly Order (Manual Soldering)

Always solder in order from smallest/lowest components to largest to avoid blocking access:

- SMD resistors and capacitors (0402/0603) — use solder paste + hot air or iron

- AMS1117-3.3 SOT-223

- HLW8012 / BL0937 SOP-8

- PC817 DIP-4 optocouplers

- S8050 TO-92 transistors

- 1N4007 diodes

- Fuse holder

- HLK-PM01 module (SIP-4 pins — through hole)

- HF3FF relay(s)

- Screw terminal blocks

- ESP32-C6 SuperMini (last — stamp hole pads, solder edges carefully)

- Expansion header

## 8.3 Test Sequence

⚠️ NEVER connect mains power without a fuse and an isolation transformer during testing.

| Test Step | Expected Result |
| --- | --- |
| 1. Visual inspection | No solder bridges, all components correctly oriented |
| 2. Continuity check (unpowered) | No short between 5V and GND. No short between LIVE and NEUTRAL |
| 3. Power-on via USB-C only (no mains) | 3.3V rail = 3.3V, ESP32 boots, serial output visible |
| 4. Flash firmware via USB | ESPHome flashes successfully, WiFi connects |
| 5. Relay test (USB power, no mains) | GPIO toggle → relay click heard, contacts verified with meter |
| 6. Mains power test (via isolation transformer) | 5V rail = 5.0–5.2V, HLK-PM01 operating |
| 7. Energy monitor calibration | Compare readings with reference clamp meter, adjust calibration constants |
| 8. Load test | Connect 100W bulb, switch on/off 100× times, check relay contact reliability |
| 9. Temperature test | Run 2 hours at full load, check component temperatures |
| 10. HomeAssistant integration | Device discovered, relay control and energy sensors visible in HA |

# 9. Quick Reference Cards

## 9.1 GPIO Pin Assignment Summary

| GPIO | Function | Notes |
| --- | --- | --- |
| GPIO0 | Relay CH1 | Active-low, inverted: true in ESPHome |
| GPIO1 | Relay CH2 | Active-low, inverted: true |
| GPIO2 | Relay CH3 (expansion) | Via expansion header |
| GPIO3 | Relay CH4 (expansion) | Via expansion header |
| GPIO20 | HLW8012 CF | Power pulse input |
| GPIO21 | HLW8012 CF1 | V/I pulse input |
| GPIO22 | HLW8012 SEL | Mode select |
| GPIO8 | WS2812 LED | Onboard RGB on SuperMini |
| GPIO9 | Boot / Button | Pull-up, active-low boot signal |
| GPIO4 | I2C SDA (optional) | For OLED/display expansion |
| GPIO5 | I2C SCL (optional) | For OLED/display expansion |
| GPIO19/20 | USB D-/D+ | Reserved for USB, do not use as GPIO |

## 9.2 Component Substitutions

| Original Part | Acceptable Substitutes |
| --- | --- |
| PC817 | EL817 (drop-in), 4N35 (adjust resistor), TLP521 (drop-in) |
| S8050 NPN | 2N2222A, BC337, SS8050 — same TO-92, Ic>300mA |
| 1N4007 | 1N4001–1N4006, FR107, UF4007 |
| HLW8012 | BL0937 (exact drop-in, same ESPHome driver) |
| AMS1117-3.3 | LM1117-3.3, SPX3819-3.3, LD1117-3.3 |
| HLK-PM01 (5V) | HLK-PM03, Hi-Link HLK-5M05 (5V/1A, for 4+ relays) |
| HF3FF/005-1ZST | JQC-3FF-S-Z (same footprint), SRD-05VDC-SL-C |

## 9.3 HF3FF Relay Pinout (Top View, DIP-5)

| Pin | Function |
| --- | --- |
| Pin 1 | Coil (+) — connect to S8050 Collector |
| Pin 2 | Coil (–) — connect to 5V rail |
| Pin 3 | NO (Normally Open contact) |
| Pin 4 | COM (Common contact) |
| Pin 5 | NC (Normally Closed contact) |

📐 RELAY BODY DIMENSIONS: 19.0mm (L) × 15.2mm (W) × 15.5mm (H). Pin pitch: 3.5mm. Row spacing: 7.5mm. Use these for your KiCad footprint if not available from library.

## 9.4 KiCad Footprint Sources for All Components

| Component | KiCad Source |
| --- | --- |
| ESP32-C6 SuperMini | Create manually (see Step-by-Step Footprint Guide) or search GitHub |
| HF3FF Relay | SnapEDA: search 'HF3FF/005-1ZST' → download KiCad format |
| HLK-PM01 | GitHub: search 'HLK-PM01 kicad' — multiple community repos available |
| PC817 | KiCad built-in library: Opto_Coupler_NEC_PC817 |
| AMS1117-3.3 | KiCad built-in: Regulator_Linear:AMS1117-3.3 |
| HLW8012 | KiCad built-in: RF_Module or search Mouser/SnapEDA |
| 1N4007 | KiCad built-in: Diode_THT:D_DO-41 |
| S8050 | KiCad built-in: Transistor_THT:TO-92 |
| KF350 terminals | KiCad built-in: Connector_PinHeader_2.54mm series |

— End of Document —

This guide covers design, layout, firmware, safety, and production for the ESP32-C6 SuperMini Smart Relay PCB platform.