MGM240LD22VIF2 Schematic Design Guide

CM5 MINIMA Carrier Board - Zigbee / Thread / Matter Integration

Based on Silicon Labs MGM240L Datasheet Rev 1.1 | SPI Interface | Multi-PAN RCP Firmware

# 1. Module Overview

The MGM240LD22VIF2 is a multiprotocol wireless module from Silicon Labs based on the EFR32MG24 SoC. It operates as a Radio Co-Processor (RCP) connected to the CM5 via SPI, running Silicon Labs Multi-PAN RCP firmware that enables simultaneous Zigbee coordinator and Thread border router operation.

| Parameter | Value |
| --- | --- |
| Part Number | MGM240LD22VIF2 |
| Protocols | Zigbee 3.0, OpenThread, Matter-ready, BLE 5.4, Bluetooth Mesh |
| Flash / RAM | 1536 kB Flash / 256 kB RAM |
| Secure Vault | HIGH - Secure Boot, Anti-Tamper, DPA Countermeasures, PUF Key Storage |
| AI/ML Engine | Matrix Vector Processor (MVP) - Yes |
| TX Power | 10 dBm max |
| RX Sensitivity | -105.8 dBm @ 802.15.4 ¦ -98.2 dBm @ BLE 1 Mbps |
| Supply Voltage | 1.8V to 3.8V (nominal 3.0V) - connect to +3V3_PI |
| Antenna | Built-in PCB Meandered Inverted-F Antenna (MIFA) - no external antenna needed |
| Dimensions | 15.5 mm x 22.5 mm |
| GPIO Pins | 12 GPIO pins available |
| Interface to CM5 | SPI (USART0 on PC00-PC03) + IRQ + RESET + BOOT |
| Certifications | CE, FCC, ISED, MIC, SRRC, KC, NCC, ACMA + 17 more countries |
| Lifecycle | Guaranteed until at least June 2033 |

# 2. MGM240L Pin Definitions

The MGM240L has 18 pads total on its bottom side - pins 1-12 on the long sides and pins 13-18 only accessible when placed horizontally. All pins used in this design are accessible in horizontal placement.

## 2.1 Complete Pin Table

| Pin | Name | Direction | Connect To | Notes |
| --- | --- | --- | --- | --- |
| 1 | VDD | Power IN | +3V3_PI | Main supply 1.8-3.8V. NO external filtering needed (internal LPF). Connect directly. |
| 2 | GND | GND | GND | Ground. Connect all GND pads to solid ground plane. |
| 3 | GND | GND | GND | Ground. |
| 4 | GND | GND | GND | Ground. |
| 5 | DEC | NC | Leave NC | Internal decoupling. Leave DISCONNECTED per datasheet. |
| 6 | PC00/SCLK | Input | ZB_CLK (GPIO21) | SPI Clock from CM5. USART0 CLK function. |
| 7 | PC01/MOSI | Input | ZB_MOSI (GPIO20) | SPI MOSI from CM5. USART0 TX function. |
| 8 | PC02/MISO | Output | ZB_MISO (GPIO19) | SPI MISO to CM5. USART0 RX function. |
| 9 | PC03/CS | Input | ZB_CS (GPIO18) | SPI Chip Select. Active LOW. USART0 CS function. |
| 10 | PA00/nRESET | Input | ZB_RESET (GPIO17) | Active LOW reset. Has internal pull-up to VDD. Do NOT add external pull-up per datasheet. |
| 11 | PB01/IRQ | Output | ZB_IRQ (GPIO26) | Interrupt to CM5. Active LOW. Add 10k pull-up to 3.3V. |
| 12 | PA04/BOOT | Input | ZB_BOOT (GPIO27) | Firmware update boot mode. Add 10k pull-up to 3.3V. Pull LOW to enter bootloader. |
| 13 | PB00/LED | Output | LED Circuit | Zigbee/Thread status LED. Drive via 330 ohm resistor to LED. |
| 14 | PA02/SWDCK | Input | Debug Header | SWD Clock for programming/debugging. Connect to debug header. |
| 15 | PA03/SWDIO | Bidir | Debug Header | SWD Data I/O. Connect to debug header. |
| 16 | PA01/PTI_DATA | Output | NC or Debug | Packet Trace Interface. Optional. Leave NC if debug header not used. |
| 17 | GND (pad) | GND | GND | Large thermal/ground pad on bottom. Must connect to ground plane via via stitching. |
| 18 | GND (shield) | GND | GND | RF shield GND tabs on sides. Connect to ground plane. |

# 3. Complete Schematic Design

## 3.1 Power Supply Circuit

The MGM240L requires only a single 3.3V supply. Do NOT add external filtering capacitors - the datasheet explicitly states this may cause voltage drops below minimum during TX bursts.

| Net/Signal | Connection | Component | Value | Notes |
| --- | --- | --- | --- | --- |
| VDD (Pin 1) | +3V3_PI | Direct wire | - | NO external caps. Module has internal filtering. |
| GND (Pins 2,3,4) | GND | Via stitching | - | Multiple GND connections to solid pour. |
| DEC (Pin 5) | NC | No connection | - | Leave floating. Critical - do not connect. |
| Thermal Pad | GND plane | 4x vias min | - | Place 4 vias under module pad for thermal/GND. |

## 3.2 SPI Interface to CM5

SPI runs between the MGM240L (USART0) and CM5 GPIO18-21. The SPI nets in your existing schematic (MOSI_GPIO20, MISO_GPIO19, SCLK_GPIO21, CS_GPIO18) will be renamed with ZB_ prefix on the new Zigbee sheet and connected via global net labels.

| MGM240L Pin | MGM240L Signal | Net Label | CM5 GPIO | Direction |
| --- | --- | --- | --- | --- |
| Pin 6 | PC00 / SCLK | ZB_CLK | GPIO21 | CM5 Output -> MGM240L |
| Pin 7 | PC01 / MOSI | ZB_MOSI | GPIO20 | CM5 Output -> MGM240L |
| Pin 8 | PC02 / MISO | ZB_MISO | GPIO19 | MGM240L Output -> CM5 |
| Pin 9 | PC03 / CS | ZB_CS | GPIO18 | CM5 Output -> MGM240L (Active LOW) |

NOTE: SPI bus speed: Use up to 8 MHz for Multi-PAN RCP firmware. The CM5 SPI controller handles this natively.

## 3.3 Control Signals

| MGM240L Pin | Signal | Net Label | CM5 GPIO | Pull-up / Notes |
| --- | --- | --- | --- | --- |
| Pin 10 | PA00 / nRESET | ZB_RESET | GPIO17 | NO external pull-up. Internal pull-up only. CM5 drives LOW to reset. |
| Pin 11 | PB01 / IRQ | ZB_IRQ | GPIO26 | R_ZB1: 10k pull-up to +3V3_PI. Active LOW interrupt. |
| Pin 12 | PA04 / BOOT | ZB_BOOT | GPIO27 | R_ZB2: 10k pull-up to +3V3_PI. CM5 pulls LOW to enter UART bootloader for firmware flash. |

## 3.4 Status LED Circuit

A dual-color LED (green/blue) is recommended to show Zigbee vs Thread activity. Alternatively use a single green LED for simplicity.

| Component | Value | Net | Connects To | Notes |
| --- | --- | --- | --- | --- |
| R_LED1 | 330 ohm 0402 | ZB_LED | MGM240L Pin 13 | Current limiting resistor. 3.3V / 330 = ~10mA. |
| LED_ZB | Green LED 0402 | ZB_LED -> GND | Anode to R_LED1, Cathode to GND | Green = Zigbee/Thread active. Place near board edge for visibility. |

## 3.5 SWD Debug Header

A 4-pin 1.27mm pitch header (or test points) for firmware programming and debugging with J-Link or Silicon Labs debugger. Recommended for prototype - can be removed for final product.

| Header Pin | Signal | MGM240L Pin | Notes |
| --- | --- | --- | --- |
| 1 | VDD (3.3V) | +3V3_PI | Reference voltage for debugger level shifting. |
| 2 | SWDIO | Pin 15 (PA03) | Bidirectional SWD data. |
| 3 | SWDCK | Pin 14 (PA02) | SWD clock input. |
| 4 | GND | GND | Debugger ground reference. |

# 4. CM5 GPIO Assignment Changes

The following changes must be made to your existing CM5 schematic sheet. Several GPIOs currently marked with X (no-connect) need to be re-assigned.

## 4.1 Existing IO Header - Signal Redirect

The SPI signals (GPIO18-21) in your existing schematic go to the IO header connector. Since we are dedicating these to the MGM240L, they will be removed from the IO header. Update the IO sheet accordingly.

| CM5 GPIO | Old Net Label | New Net Label | Action |
| --- | --- | --- | --- |
| GPIO21 | SCLK_GPIO21 | ZB_CLK | Remove from IO header. Connect to new ZB_CLK net. |
| GPIO20 | MOSI_GPIO20 | ZB_MOSI | Remove from IO header. Connect to new ZB_MOSI net. |
| GPIO19 | MISO_GPIO19 | ZB_MISO | Remove from IO header. Connect to new ZB_MISO net. |
| GPIO18 | CS_GPIO18 | ZB_CS | Remove from IO header. Connect to new ZB_CS net. |
| GPIO17 | X (no-connect) | ZB_RESET | Remove X marker. Connect to ZB_RESET net for MGM240L nRESET. |
| GPIO26 | X (no-connect) | ZB_IRQ | Remove X marker. Connect to ZB_IRQ net for MGM240L interrupt. |
| GPIO27 | RST_GPIO27 | ZB_BOOT | Was unused RST. Repurpose to ZB_BOOT for MGM240L firmware boot. |

NOTE: After this change, the IO header (MOSI, MISO, SCLK, CS, BL, RST, DC labels) will lose SPI signals. If you need SPI on the expansion header, add GPIO16 as a second CS and use GPIO6/13 for an alternate SPI bus. For now, dedicating GPIO18-21 to Zigbee is the cleaner solution.

# 5. New KiCad Schematic Sheet - Zigbee_Thread

## 5.1 Sheet Creation

- In KiCad root schematic: Place > Add Hierarchical Sheet

- Sheet name: Zigbee_Thread

- Filename: zigbee_thread.kicad_sch

- Sheet number: 9 (or next available)

## 5.2 Hierarchical Port Labels on New Sheet

These are the interface signals between the Zigbee sheet and the rest of the schematic. Add them as Hierarchical Labels (Place > Add Hierarchical Label):

| Label Name | Type | Connects To (on CM5 sheet) |
| --- | --- | --- |
| ZB_CLK | Input | ZB_CLK <- GPIO21 |
| ZB_MOSI | Input | ZB_MOSI <- GPIO20 |
| ZB_MISO | Output | ZB_MISO -> GPIO19 |
| ZB_CS | Input | ZB_CS <- GPIO18 |
| ZB_RESET | Input | ZB_RESET <- GPIO17 |
| ZB_IRQ | Output | ZB_IRQ -> GPIO26 |
| ZB_BOOT | Input | ZB_BOOT <- GPIO27 |
| +3V3_PI | Power IN | +3V3_PI power net from main sheet |
| GND | Power IN | GND |

## 5.3 Complete Component List for Zigbee Sheet

| Ref Des | Component | Value | Package | Notes |
| --- | --- | --- | --- | --- |
| U_ZB | MGM240LD22VIF2 | - | 15.5x22.5mm custom | Main Zigbee/Thread module. |
| R_ZB1 | Pull-up resistor | 10k 0402 | 0402 | IRQ pull-up to +3V3_PI. |
| R_ZB2 | Pull-up resistor | 10k 0402 | 0402 | BOOT pull-up to +3V3_PI. |
| R_LED1 | LED resistor | 330 ohm 0402 | 0402 | Status LED current limit. |
| LED_ZB | Green LED | Green 0402 | 0402 | Zigbee/Thread status indicator. |
| H_ZB | SWD debug header | 1x4 1.27mm | 1x4P 1.27mm | Programming header. Optional for production. |
| TP_ZB1 | Test point | ZB_CLK | TP 1.0mm | Optional. Helps with debugging. |
| TP_ZB2 | Test point | ZB_MOSI | TP 1.0mm | Optional. |
| TP_ZB3 | Test point | ZB_MISO | TP 1.0mm | Optional. |
| TP_ZB4 | Test point | ZB_IRQ | TP 1.0mm | Optional. |

# 6. PCB Layout Rules - Critical for RF Performance

## 6.1 Module Placement

- Place MGM240L at the board edge - the PCB antenna extends beyond or flush with the board outline.

- Antenna end (pin 17/18 side) must face open air - never toward HDMI, USB, or other metal.

- Orient horizontally on the PCB for access to all 18 pads.

## 6.2 Antenna Keepout Zone

- NO copper on any layer within 3mm of the antenna area (inverted-F trace at one end).

- NO components within the keepout zone.

- NO ground plane under the antenna - critical for antenna efficiency.

- In KiCad: add a Fab.Courtyard or User layer keepout zone and mark it No Copper.

NOTE: The module datasheet Section 7.2 shows exact keepout dimensions. The antenna is at the END of the module (opposite side from pins 1-4). Reference Figure 7.1 in the MGM240L datasheet.

## 6.3 SPI Trace Routing

- Keep SPI traces (ZB_CLK, ZB_MOSI, ZB_MISO, ZB_CS) as short as possible - under 50mm ideal.

- Route all SPI signals on the same layer.

- Add ground pour around SPI traces to reduce EMI.

- Matched length routing is NOT needed at 8 MHz SPI speeds.

## 6.4 Ground Via Stitching

- Place minimum 4 vias under the module thermal pad connecting top copper to inner GND plane.

- Stitch ground plane around the module perimeter with vias spaced 3-5mm apart.

- This is critical for RF ground reference and thermal dissipation.

# 7. Firmware - Multi-PAN RCP Setup

## 7.1 What Firmware to Flash

The MGM240LD22VIF2 runs as an RCP (Radio Co-Processor). The CM5 runs the actual Zigbee/Thread stack software. You need the Multi-PAN RCP firmware which allows simultaneous Zigbee + Thread on one chip.

| Item | Detail |
| --- | --- |
| Firmware | Multi-PAN RCP from Silicon Labs GSDK (Gecko SDK) |
| SDK | Silicon Labs Simplicity Studio + GSDK v4.x |
| Project template | RCP Multi-PAN (select EFR32MG24 target) |
| SPI configuration | USART0: CLK=PC00, MOSI=PC01, MISO=PC02, CS=PC03 |
| CM5 software | zigbee2mqtt (Zigbee) + otbr-agent (Thread Border Router) |
| HA integration | ZHA or Zigbee2MQTT addon + Thread addon in Home Assistant |

## 7.2 Flashing Procedure

- Pull ZB_BOOT (PA04) LOW via CM5 GPIO27 before power-on or reset

- Module boots into UART XMODEM bootloader mode (pre-programmed from factory)

- Alternatively use J-Link via the H_ZB SWD debug header

- Flash Multi-PAN RCP .hex firmware using Simplicity Commander or J-Flash

- Release BOOT pin, reset module via ZB_RESET - RCP boots and awaits SPI commands from CM5

# 8. Schematic Completion Checklist

- [ ] Add new hierarchical sheet Zigbee_Thread in root schematic

- [ ] Download MGM240LD22VIF2 symbol + footprint from SnapEDA or create manually

- [ ] Place MGM240LD22VIF2 (U_ZB) on new sheet

- [ ] Connect VDD pin to +3V3_PI - NO filtering caps

- [ ] Leave DEC pin unconnected

- [ ] Connect GND pins 2,3,4 and shield tabs to GND

- [ ] Add SPI net labels ZB_CLK, ZB_MOSI, ZB_MISO, ZB_CS

- [ ] Add control net labels ZB_RESET, ZB_IRQ, ZB_BOOT

- [ ] Add R_ZB1 10k pull-up on ZB_IRQ to +3V3_PI

- [ ] Add R_ZB2 10k pull-up on ZB_BOOT to +3V3_PI

- [ ] NO pull-up on nRESET - datasheet prohibits it

- [ ] Add R_LED1 330 ohm + LED_ZB green LED on PB00 (Pin 13)

- [ ] Add H_ZB 4-pin 1.27mm SWD header (VDD, SWDIO, SWDCK, GND)

- [ ] Update CM5 sheet - remove X markers from GPIO17, GPIO26, GPIO27

- [ ] Update CM5 sheet - rename GPIO18-21 nets to ZB_CS, ZB_MISO, ZB_MOSI, ZB_CLK

- [ ] Update IO sheet - remove MOSI, MISO, SCLK, CS from expansion header

- [ ] Run ERC (Electrical Rules Check) - resolve all errors

- [ ] Run DRC (Design Rules Check) after layout with antenna keepout zone

Reference: MGM240L Wireless Gecko Multiprotocol Lighting Module Datasheet Rev 1.1 | Silicon Labs GSDK Documentation | CM5 MINIMA Schematic Rev 3.1