MGM240LD22VIF2 PCB Layout Guide

CM5 Home Automation Hub — Zigbee/Thread/Matter Module

KiCad PCB Layout — Step by Step

# 1. Before You Start — KiCad Setup

## 1.1 Update PCB from Schematic

- Open KiCad PCB Editor (.kicad_pcb file)

- Go to Tools → Update PCB from Schematic (F8)

- Click Update PCB — all components will appear in the center

- Press Escape, then arrange components manually

## 1.2 Set Design Rules (DRC Rules)

Go to File → Board Setup → Design Rules and set:

| Parameter | Value | Reason |
| --- | --- | --- |
| Min trace width | 0.15mm | Standard PCB fab minimum |
| Min clearance | 0.15mm | Standard PCB fab minimum |
| SPI signal traces | 0.2mm preferred | Low impedance, clean signal |
| Power traces (3.3V) | 0.5mm minimum | Module draws up to 30mA TX |
| Via drill size | 0.3mm minimum | Standard fab capability |
| Via annular ring | 0.15mm minimum | Standard fab capability |

# 2. MGM240L Module Placement — Most Critical Step

🚫  This is the MOST IMPORTANT step. Wrong placement = poor RF performance. Cannot be fixed after PCB manufacture.

## 2.1 Where to Place the Module

The MGM240L MUST be placed at the EDGE of your PCB with the antenna end hanging over or flush with the board edge.

| Rule | Value | From Datasheet |
| --- | --- | --- |
| Module position | At PCB edge | Section 7.1 |
| Antenna direction | Facing away from board / open air | Figure 7.2 |
| Copper clearance under antenna | 12.5mm wide, 5mm deep — NO copper at all | Figure 7.1 |
| GND pour width alongside module | 12.5mm minimum | Section 7.1 |
| Board thickness (optimal) | 0.8mm for best antenna efficiency | Section 7.2 |
| Keep components away from antenna | 5mm minimum clearance | Figure 7.2 |

## 2.2 Step-by-Step Placement in KiCad

- Click on U2 (MGM240L) and press E to open properties

- Set Position X and Y to place it at your chosen board edge

- Rotate if needed — antenna end (top of module, pins 13-18 side) must face outward

- Lock the component: Right-click → Lock

## 2.3 Create Antenna Keepout Zone

- In PCB editor, go to Place → Rule Area (or press Z)

- Draw a rectangle covering 12.5mm × 5mm at the antenna end

- In the Rule Area dialog, check ALL of these: No Copper, No Tracks, No Vias, No Components

- Set it on ALL copper layers (F.Cu, B.Cu, and all inner layers)

- This zone is NON-NEGOTIABLE — violating it will destroy RF performance

🚫  NEVER place traces, vias, copper pour, or components under the antenna area. This permanently degrades RF range.

# 3. PCB Layer Stack Recommendation

For your CM5 carrier board with RF module, a 4-layer stack is recommended:

| Layer | Name | Purpose |
| --- | --- | --- |
| Layer 1 (Top) | F.Cu | Signal routing — SPI, GPIO, most traces |
| Layer 2 | In1.Cu | Solid GND plane — continuous, no cuts under module |
| Layer 3 | In2.Cu | Power planes — 3.3V, 5V areas |
| Layer 4 (Bottom) | B.Cu | Additional signals, less RF-sensitive traces |

✅  GND plane on Layer 2 directly under the module (except antenna area) provides excellent RF shielding and return path.

# 4. SPI Signal Routing — Step by Step

## 4.1 SPI Routing Rules

| Signal | GPIO | Max Length | Trace Width | Priority |
| --- | --- | --- | --- | --- |
| ZB_CLK (SCLK) | GPIO21 → PC00 | 50mm max | 0.2mm | HIGHEST |
| ZB_MOSI (MOSI) | GPIO20 → PC01 | 50mm max | 0.2mm | HIGH |
| ZB_MISO (MISO) | GPIO19 → PC02 | 50mm max | 0.2mm | HIGH |
| ZB_CS (CS) | GPIO18 → PC03 | 50mm max | 0.2mm | HIGH |
| ZB_RESET | GPIO27 → RST | 100mm ok | 0.2mm | NORMAL |
| ZB_IRQ | GPIO26 → PD00 | 100mm ok | 0.2mm | NORMAL |
| BOOT_GPIO17 | GPIO17 → PA04 | 100mm ok | 0.2mm | LOW |

## 4.2 Step-by-Step SPI Routing in KiCad

- Press X to start routing traces

- Route all 4 SPI signals (CLK, MOSI, MISO, CS) on the SAME layer — F.Cu preferred

- Keep traces parallel and close together — avoid crossing other signals

- Keep total trace length under 50mm for each SPI signal

- Do NOT route SPI traces under the antenna keepout zone

- Add ground pour on F.Cu around SPI traces after routing all signals

⚠️  Never route SPI clock (ZB_CLK) parallel to antenna — keep at least 3mm separation from antenna area.

# 5. Power and GND Connections

## 5.1 VDD (Pin 13) — Power Supply

| Item | Requirement |
| --- | --- |
| Supply voltage | 3.3V from CM5 +3V3_PI rail |
| Trace width to VDD | 0.5mm minimum — module draws up to 30mA TX |
| External capacitors | NOT required — module has internal filtering |
| External filtering | NOT recommended — may cause voltage drops during TX |
| Connection point | Short direct trace from 3.3V plane to pin 13 |

🚫  Do NOT add external decoupling capacitors on VDD. The datasheet explicitly states additional external filtering is NOT recommended.

## 5.2 GND (Pins 6 and 14)

- Both GND pins must connect to solid GND plane on Layer 2

- Place 2-3 vias directly adjacent to pin 6 and pin 14 pads going to Layer 2 GND plane

- Via size: 0.6mm outer diameter, 0.3mm drill — standard

- This provides low-inductance GND return path for RF currents

## 5.3 VDD Test Point Keepout

The datasheet (Section 8.2, Note 3) mentions a VDD test point on the bottom of the module at coordinates X=2.65mm, Y=6.45mm.

⚠️  Keep a 1.4mm × 1.4mm copper-free area around the VDD test point on your PCB to prevent accidental short circuit.

# 6. SWD Debug Header Placement

The SWD debug header (J_SWD — Conn_01x04_Pin) is used for flashing firmware onto the MGM240L using a J-Link or similar debugger.

## 6.1 Placement Rules

- Place J_SWD close to the MGM240L module — within 30mm

- Make it accessible on the PCB edge or near a corner

- 1.27mm pitch — standard for embedded debug headers

- Keep SWD trace lengths under 100mm

## 6.2 SWD Connections

| Header Pin | Signal | MGM240L Pin | Trace Width |
| --- | --- | --- | --- |
| Pin 1 | +3V3_PI (VCC) | Power only — no MGM240L pin | 0.5mm |
| Pin 2 | SWDIO | PA02 (Pin 2) | 0.2mm |
| Pin 3 | SWCLK | PA01 (Pin 1) | 0.2mm |
| Pin 4 | GND | GND plane | 0.5mm |

# 7. Ground Pour and Copper Fill

## 7.1 F.Cu (Top Layer) Ground Pour

- After routing all traces, go to Place → Zone (or press Z)

- Draw a zone covering the entire board area

- Set Net = GND, Layer = F.Cu

- Set Clearance = 0.3mm, Min width = 0.2mm

- Press B to fill all zones

- The GND pour automatically avoids traces and pads

⚠️  The GND pour must NOT enter the antenna keepout zone. KiCad will respect the Rule Area you created in Step 2.3.

## 7.2 Via Stitching Around Module

- Add GND vias around the perimeter of the MGM240L footprint

- Spacing: one via every 3-5mm

- This connects F.Cu GND pour to Layer 2 GND plane

- Improves RF ground return path and reduces EMI

# 8. LED Indicator (ZB_LED)

The status LED circuit (PB00 → 330Ω → LED → GND) is an output from the MGM240L.

- Place the LED and resistor near the module — within 20mm

- LED resistor R_LED: 0402 size, 330Ω

- LED: 0402 size, green

- Trace from PB00 to resistor: 0.2mm width

- Place LED on board edge or near enclosure window if building into housing

# 9. DRC — Design Rule Check

After completing all routing, run DRC to catch errors before sending to fab.

## 9.1 Run DRC

- Go to Inspect → Design Rules Checker

- Click Run DRC

- Fix all ERROR level issues — these will cause PCB failures

- Review WARNING level issues — most are acceptable

## 9.2 Critical Items to Check Manually

| Check Item | What to Verify | Pass/Fail Criteria |
| --- | --- | --- |
| Antenna keepout | No copper, traces, or vias in keepout zone | Zone must be completely empty |
| VDD test point | 1.4×1.4mm keepout area clear on bottom layer | No copper overlap |
| SPI trace lengths | Measure each SPI trace — CLK, MOSI, MISO, CS | All under 50mm |
| GND via stitching | Vias around module perimeter | At least one per 5mm |
| GND pin connections | Both pin 6 and pin 14 have vias to GND plane | 2+ vias each |
| DEC pin (pin 5) | No connection — must be floating | No trace connected |
| DNC pin (pin 12) | No connection — must be floating | No trace connected |

# 10. Fabrication Files (Gerber Export)

When PCB layout is complete and DRC passes, export Gerber files for manufacturing.

- Go to File → Fabrication Outputs → Gerbers (.gbr)

- Select all layers: F.Cu, B.Cu, In1.Cu, In2.Cu, F.Mask, B.Mask, F.Silkscreen, Edge.Cuts

- Also export Drill files: File → Fabrication Outputs → Drill Files

- Zip all files together and upload to your PCB fab (JLCPCB, PCBWay, etc.)

## Recommended Fab Specifications

| Parameter | Recommended Value | Reason |
| --- | --- | --- |
| Layers | 4 | GND plane required for RF |
| Board thickness | 0.8mm | Optimal for MGM240L antenna (Section 7.2 datasheet) |
| Copper weight | 1oz outer, 0.5oz inner | Standard |
| Surface finish | HASL or ENIG | ENIG better for fine-pitch pads |
| Solder mask | Both sides | Standard |
| Min trace/space | 0.15/0.15mm | Matches design rules |

# 11. Complete PCB Layout Checklist

| # | Task | Status |
| --- | --- | --- |
| 1 | MGM240L placed at PCB edge with antenna facing outward | ☐ |
| 2 | Antenna keepout zone created — 12.5mm × 5mm, all layers | ☐ |
| 3 | No copper/traces/vias in antenna keepout zone | ☐ |
| 4 | SPI traces (CLK, MOSI, MISO, CS) all on same layer, under 50mm | ☐ |
| 5 | GND pour added on F.Cu (top layer) | ☐ |
| 6 | GND via stitching around module perimeter (3-5mm spacing) | ☐ |
| 7 | GND vias at pin 6 and pin 14 to Layer 2 GND plane | ☐ |
| 8 | VDD trace 0.5mm width from 3.3V to pin 13 | ☐ |
| 9 | No external capacitors on VDD pin 13 | ☐ |
| 10 | VDD test point keepout 1.4×1.4mm on bottom layer | ☐ |
| 11 | DEC (pin 5) floating — no connection | ☐ |
| 12 | DNC (pin 12) floating — no connection | ☐ |
| 13 | SWD header placed near module, accessible | ☐ |
| 14 | LED + 330Ω resistor placed near module | ☐ |
| 15 | DRC run — all errors resolved | ☐ |
| 16 | Gerber files exported and verified | ☐ |

Reference: MGM240L Datasheet Rev 1.0 | Silicon Laboratories | CM5 Home Automation Hub Project