Smart Relay Switch Module

Complete Engineering & Build Guide

KiCad Project: RelaySwitch_C6  |  Target: 45×50mm  |  2-Channel + Wall Switch Input

Aqara T1/T2 Compatible — Indian Switchboard Fitment

Document Version: 2.0  |  Date: February 2026

BUSINESS CONFIDENTIAL — For Production Use

# 1. Current Design Audit & Issues Found

After analyzing your KiCad project files (RelaySwitch_C6.kicad_sch, .kicad_pcb), the following critical issues were identified that must be fixed before production:

## 1.1 Current Board Dimensions

Measured from Edge.Cuts: 58.5mm × 54.5mm (from coordinates 131.5,55 to 190,109.5)

Target size: 45mm × 50mm (per migration guide)

Size reduction needed: ~23% area reduction — requires component migration to SMD

## 1.2 Critical Issues Found

Issue #1: Most Resistors Still THT (Through-Hole)

R1 (330Ω), R2 (10kΩ), R3 (330Ω), R4 (470kΩ), R5 (10kΩ), R6–R9 (470kΩ), R10 (1mΩ shunt), R13 (470kΩ) — all still using THT R_Axial_DIN0207 footprint (6.3mm body). These must be converted to SMD 0402 or 0603 to achieve the 45×50mm target.

Issue #2: Capacitors C2–C6 Still THT

C2 (10µF), C3 (100nF), C4 (10µF), C5 (100nF), C6 (100nF) are all using THT C_Disc_D5.0mm footprints. These must be changed to SMD: 100nF → 0402, 10µF → 0805 ceramic MLCC.

Issue #3: All Components on Front Side Only

Every single footprint in the PCB is on F.Cu layer. The migration guide specifies a two-sided design: HV components on top, LV components on bottom. This is essential for both safety (creepage distance) and space reduction.

Issue #4: No External Wall Switch Input (S0/S1)

The current design has no provision for an external wall switch connection. This is CRITICAL for Aqara T1/T2-like functionality. The wall switch must connect to a GPIO on the ESP32-C6 so users can toggle the relay manually from the existing wall switch.

Issue #5: Varistor RV1 is THT

RV1 (275VAC MOV) uses a 7mm disc footprint (RV_Disc_D7mm). For compact design, consider an SMD varistor (like Bourns CDSOD323-T05C or Littelfuse AUMOV) or keep THT if needed for voltage rating.

Issue #6: HLW8012 Power Metering Resistor Network

R4, R6, R7, R8 (all 470kΩ) form the voltage divider for HLW8012. R13 (470kΩ) is also part of this. R10 (1mΩ) is the current shunt. These must be precisely positioned near U5 (HLW8012) for accurate measurement. The shunt resistor R10 needs proper Kelvin connection.

Issue #7: Missing Switch Button Connector/Pads

No connector or pads exist for S0/S1 (external wall switch). Must add a 2-pin or 3-pin connector for wall switch input, similar to Aqara's S0/S1 terminals.

# 2. Relay Recommendation: Omron G5Q-1A DC5

Your current design already uses the Omron G5Q-1 footprint (Relay_THT:Relay_SPDT_Omron-G5Q-1), which is an excellent choice. Here is the detailed analysis:

## 2.1 Why Omron G5Q-1A DC5 is the Best Choice

| Parameter | Omron G5Q-1A DC5 | HF3FF/005-1ZST (Original) | HK4100F-DC5V |
| --- | --- | --- | --- |
| Contact Form | SPST-NO (Form A) | SPDT (Form C) | SPDT (Form C) |
| Contact Rating | 10A @ 250VAC | 10A @ 250VAC | 3A @ 250VAC |
| Coil Voltage | 5VDC | 5VDC | 5VDC |
| Coil Resistance | 125Ω | 70Ω | 125Ω |
| Coil Current | 40mA | 71mA | 40mA |
| Body Size | 12.5 × 7.5 × 10mm | 18.4 × 10.2 × 15mm | 12 × 7.2 × 10mm |
| PCB Footprint | ~12.5 × 7.5mm | ~18 × 10mm | ~12 × 7mm |
| Electrical Life | 100,000 ops @ 10A | 100,000 ops | 100,000 ops @ 3A |
| Safety Certifications | UL, TÜV, CQC | UL, TÜV | UL, CQC |
| Availability (India) | Excellent (Mouser/Digikey) | Limited | Excellent (local) |
| Price (approx.) | ₹80–120 | ₹60–90 | ₹15–25 |

## 2.2 Recommendation Summary

Primary Choice: Omron G5Q-1A DC5 — 10A rating, compact 12.5×7.5mm, 40mA coil (half of HF3FF), UL/TÜV/CQC certified, excellent for Indian 230V market

Budget Alternative: HK4100F-DC5V-SHG — If cost is primary concern and load is ≤3A (LED lights only). NOT recommended for fans or heaters.

Premium Alternative: Omron G5Q-1A-EL DC5 — Enhanced life version (100K ops @ 10A guaranteed), slightly higher cost but better for commercial product

⚠ WARNING: For the Indian market at 230VAC, ALWAYS use a relay rated ≥10A with proper safety certifications (UL/TÜV/CQC/BIS). The HK4100F at 3A is NOT suitable for general-purpose wall switch modules.

ℹ NOTE: Your KiCad project already has the correct footprint assigned: Relay_THT:Relay_SPDT_Omron-G5Q-1. The G5Q-1A (SPST-NO) uses the SAME footprint — just leave pins 3 (NC contact) unconnected.

# 3. Adding External Wall Switch Input (Aqara T1/T2 Style)

## 3.1 How Aqara T1/T2 Wall Switch Works

The Aqara T1 module has terminals labeled S0 and S1. An external wall switch (rocker or momentary) connects between these two terminals. When the switch state changes, the module's MCU detects it and toggles the relay accordingly. The key design principle: the wall switch operates on the LOW VOLTAGE side (3.3V logic), completely isolated from mains.

## 3.2 Circuit Design for Wall Switch Input

We need to connect 1-2 external wall switches to the XIAO ESP32-C6 GPIO pins. The ESP32-C6 on the XIAO board has internal pull-ups, but for noise immunity in a mains environment, we MUST add external components.

Circuit for Each Switch Channel (SW1 for K1, SW2 for K2):

External Switch Terminal: 2-pin screw terminal (3.5mm pitch) — same as your load connectors

Connection: One terminal to GPIO pin, other terminal to GND

Pull-up Resistor: 10kΩ to +3V3 (keeps GPIO HIGH when switch is open)

Filter Capacitor: 100nF ceramic from GPIO to GND (debounce + noise filter)

Series Resistor: 1kΩ in series with GPIO (ESD protection)

Clamping Diode: Optional BAT54S or 1N4148W — clamps voltage to 3.3V rail

Schematic Description (Text Representation):

+3V3 ──── R_pullup (10kΩ) ──┬── R_series (1kΩ) ──── GPIO_SWx (ESP32-C6)

│

├── C_filter (100nF) ── GND

│

Switch Terminal S0

│

Switch Terminal S1 ── GND

GPIO Pin Assignment on XIAO ESP32-C6:

| Function | XIAO Pin Label | ESP32-C6 GPIO | Notes |
| --- | --- | --- | --- |
| Switch 1 (for K1) | D0 (GPIO2) | GPIO2 | External pull-up + RC filter |
| Switch 2 (for K2) | D1 (GPIO3) | GPIO3 | External pull-up + RC filter |
| Relay 1 Drive (K1) | Via optocoupler U3 | GPIO already assigned | Through R1 → U3 → Q1 → K1 |
| Relay 2 Drive (K2) | Via optocoupler U4 | GPIO already assigned | Through R3 → U4 → Q2 → K2 |
| HLW8012 CF | Existing connection | Existing | Power measurement pulse |
| HLW8012 CF1 | Existing connection | Existing | Power measurement pulse |
| HLW8012 SEL | Existing connection | Existing | V/I channel select |

## 3.3 Step-by-Step KiCad Schematic Modifications

Step 1: Open RelaySwitch_C6.kicad_sch in KiCad Schematic Editor.

Step 2: Add a new 2-pin screw terminal for Switch 1: Press A → search 'Screw_Terminal_01x02' → place it. Set Reference = J5, Value = SW1_INPUT. Assign footprint: TerminalBlock_Degson:TerminalBlock_Degson_DG250-3.5-02P_1x02_P3.50mm_45Degree

Step 3: Add a new 2-pin screw terminal for Switch 2: Press A → search 'Screw_Terminal_01x02' → place it. Set Reference = J6, Value = SW2_INPUT. Same footprint as above.

Step 4: Add pull-up resistors: Press A → search 'R' → place two resistors. Set R14 = 10k (footprint: Resistor_SMD:R_0402_1005Metric), R15 = 10k (same footprint). Connect R14 from +3V3 to the junction point of SW1. Connect R15 from +3V3 to the junction point of SW2.

Step 5: Add series protection resistors: Place R16 = 1k and R17 = 1k (both 0402 SMD). Connect R16 between the SW1 junction and ESP32-C6 D0 pin. Connect R17 between the SW2 junction and ESP32-C6 D1 pin.

Step 6: Add filter capacitors: Place C7 = 100nF and C8 = 100nF (both 0402 SMD). Connect C7 from the SW1 junction to GND. Connect C8 from the SW2 junction to GND.

Step 7: Wire the switch terminals: J5 Pin 1 → junction point (where R14, R16, C7 meet). J5 Pin 2 → GND. J6 Pin 1 → junction point (where R15, R17, C8 meet). J6 Pin 2 → GND.

Step 8: Add net labels: Label the GPIO connections as 'SW1_IN' and 'SW2_IN' for clarity. Add the same labels on the ESP32-C6 D0 and D1 pins.

Step 9: Run ERC: Electrical Rules Check → fix any errors. Expected: 0 errors.

ℹ NOTE: The Aqara T1 supports both rocker (bistable/toggle) and momentary (monostable/push-button) switches. Your ESPHome firmware should handle both modes: detect edge transitions (rising/falling) and toggle relay state accordingly. This is a firmware configuration, not a hardware change.

## 3.4 ESPHome Configuration for Wall Switch

Add this to your ESPHome YAML configuration for the switch inputs:

binary_sensor:

- platform: gpio

pin:

number: GPIO2

mode: INPUT_PULLUP

inverted: true

name: "Wall Switch 1"

id: wall_switch_1

filters:

- delayed_on: 50ms

- delayed_off: 50ms

on_state:

then:

- switch.toggle: relay_1

- platform: gpio

pin:

number: GPIO3

mode: INPUT_PULLUP

inverted: true

name: "Wall Switch 2"

id: wall_switch_2

filters:

- delayed_on: 50ms

- delayed_off: 50ms

on_state:

then:

- switch.toggle: relay_2

✓ TIP: For rocker switch mode (Aqara default), use 'on_state' with 'switch.toggle'. For momentary push-button mode, use 'on_press' instead. You can make this configurable via a Home Assistant select entity.

# 4. Complete Component Migration — THT to SMD

## 4.1 Resistor Migration

| Ref | Value | Current Footprint | New Footprint | SMD Size |
| --- | --- | --- | --- | --- |
| R1 | 330Ω | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0603_1608Metric | 0603 |
| R2 | 10kΩ | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0402_1005Metric | 0402 |
| R3 | 330Ω | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0603_1608Metric | 0603 |
| R4 | 470kΩ | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0402_1005Metric | 0402 |
| R5 | 10kΩ | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0402_1005Metric | 0402 |
| R6 | 470kΩ | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0402_1005Metric | 0402 |
| R7 | 470kΩ | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0402_1005Metric | 0402 |
| R8 | 470kΩ | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0402_1005Metric | 0402 |
| R9 | 1kΩ | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0402_1005Metric | 0402 |
| R10 | 1mΩ (shunt) | R_Axial_DIN0207 (THT) | Resistor_SMD:R_2512_6332Metric | 2512 |
| R11 | 1kΩ | Already R_0402 ✓ | — no change — | 0402 |
| R12 | 1kΩ | Already R_0402 ✓ | — no change — | 0402 |
| R13 | 470kΩ | R_Axial_DIN0207 (THT) | Resistor_SMD:R_0402_1005Metric | 0402 |
| R14* | 10kΩ | NEW — switch pull-up | Resistor_SMD:R_0402_1005Metric | 0402 |
| R15* | 10kΩ | NEW — switch pull-up | Resistor_SMD:R_0402_1005Metric | 0402 |
| R16* | 1kΩ | NEW — switch series | Resistor_SMD:R_0402_1005Metric | 0402 |
| R17* | 1kΩ | NEW — switch series | Resistor_SMD:R_0402_1005Metric | 0402 |

ℹ NOTE: R1 and R3 (330Ω) are optocoupler LED drive resistors carrying ~10mA. Use 0603 for better thermal handling. R10 (shunt) MUST be 2512 size for adequate power rating and 4-wire Kelvin connection.

## 4.2 Capacitor Migration

| Ref | Value | Current Footprint | New Footprint | Notes |
| --- | --- | --- | --- | --- |
| C1 | 100nF | Already C_0402 ✓ | — no change — | HLK-2M05 output decoupling |
| C2 | 10µF | C_Disc_D5.0mm (THT) | Capacitor_SMD:C_0805_2012Metric | AMS1117 input cap — use 10µF 10V X5R |
| C3 | 100nF | C_Disc_D5.0mm (THT) | Capacitor_SMD:C_0402_1005Metric | AMS1117 output decoupling |
| C4 | 10µF | C_Disc_D5.0mm (THT) | Capacitor_SMD:C_0805_2012Metric | AMS1117 output cap — use 10µF 10V X5R |
| C5 | 100nF | C_Disc_D5.0mm (THT) | Capacitor_SMD:C_0402_1005Metric | ESP32 bypass |
| C6 | 100nF | C_Disc_D5.0mm (THT) | Capacitor_SMD:C_0402_1005Metric | HLW8012 bypass |
| C7* | 100nF | NEW — switch filter | Capacitor_SMD:C_0402_1005Metric | SW1 debounce RC filter |
| C8* | 100nF | NEW — switch filter | Capacitor_SMD:C_0402_1005Metric | SW2 debounce RC filter |

⚠ WARNING: For C2 and C4, do NOT use Y5V dielectric — the capacitance drops dramatically at temperature extremes. Use X5R or X7R ceramic MLCCs rated ≥10V.

## 4.3 Other Components — Status

| Component | Status | Action Required |
| --- | --- | --- |
| U1: HLK-2M05 | ✓ Already SMD module | No change — keep XIAO_Series_Footprints:HLK-2M05 |
| U2: XIAO-ESP32-C6-DIP | ✓ DIP module (SMD castellated) | No change — keep seeed esp32:XIAO-ESP32-C6-DIP |
| U3, U4: PC817S | ✓ Already SSOP-4 | No change — keep Package_SO:SSOP-4_4.4x2.6mm_P1.27mm |
| U5: HLW8012 | ✓ Already SOIC-8 | No change — keep XIAO_Series_Footprints:SOIC8 |
| U6: AMS1117-3.3 | ✓ Already SOT-223 | No change — keep Package_TO_SOT_SMD:SOT-223-3_TabPin2 |
| Q1, Q2: MMBT8050 | ✓ Already SOT-23 | No change |
| D1, D2: 1N4007W | ✓ Already D_SMA | No change — keep Diode_SMD:D_SMA |
| K1, K2: G5Q-1-DC5 | ✓ Correct relay chosen | No change — keep Relay_THT:Relay_SPDT_Omron-G5Q-1 |
| F1: Fuse | ✓ Already 1206 SMD | No change — keep Fuse:Fuse_1206_3216Metric |
| RV1: MOV 275VAC | ⚠ THT 7mm disc | KEEP THT — no SMD MOV rated for 275VAC at this price |
| J1: AC Mains In | ✓ Correct 3.5mm pitch | No change |
| J3, J4: Load Out | ✓ Correct 3.5mm pitch | No change |
| J5*, J6*: Switch In | NEW — must add | Add TerminalBlock 3.5mm 2-pin |
| TP1-TP4: Test Points | ✓ Already THT test pads | No change |

# 5. Step-by-Step KiCad Modification Instructions

## 5.1 Schematic Changes

Phase A: Change All THT Resistor Footprints to SMD

Step 1: Open RelaySwitch_C6.kicad_sch in KiCad 9 Schematic Editor.

Step 2: Click on R1 → press E to open properties → click the Footprint field → click the browse button (...)

Step 3: In the footprint browser, search: R_0603 → select Resistor_SMD:R_0603_1608Metric → click OK

Step 4: Repeat for R3 (also 330Ω, also 0603).

Step 5: For R2, R4, R5, R6, R7, R8, R9, R13: click each → E → Footprint → browse → search R_0402 → select Resistor_SMD:R_0402_1005Metric → OK

Step 6: For R10 (1mΩ shunt): click → E → Footprint → browse → search R_2512 → select Resistor_SMD:R_2512_6332Metric → OK

Step 7: R11 and R12 are already 0402 — verify and skip.

Phase B: Change All THT Capacitor Footprints to SMD

Step 8: Click C2 → E → Footprint → browse → search C_0805 → select Capacitor_SMD:C_0805_2012Metric → OK

Step 9: Repeat for C4 (also 10µF → 0805).

Step 10: Click C3 → E → Footprint → browse → search C_0402 → select Capacitor_SMD:C_0402_1005Metric → OK

Step 11: Repeat for C5 and C6 (both 100nF → 0402).

Phase C: Add Wall Switch Input Circuits

Step 12: Place new components: 2× Screw_Terminal_01x02 (J5=SW1_INPUT, J6=SW2_INPUT), 4× R (R14=10k, R15=10k, R16=1k, R17=1k), 2× C_Small (C7=100nF, C8=100nF).

Step 13: Wire as described in Section 3.2. Use net labels SW1_IN and SW2_IN connecting to ESP32-C6 D0 and D1 pins.

Step 14: Assign footprints: J5, J6 → TerminalBlock_Degson:TerminalBlock_Degson_DG250-3.5-02P_1x02_P3.50mm_45Degree. R14-R17 → R_0402. C7, C8 → C_0402.

Phase D: Run ERC

Step 15: Go to Inspect → Electrical Rules Checker → Run ERC. Fix any errors. Common warnings from PWR_FLAG are acceptable.

Step 16: Press Ctrl+S to save the schematic.

## 5.2 PCB Layout Changes

Phase E: Sync PCB with Schematic

Step 17: Open RelaySwitch_C6.kicad_pcb in PCB Editor.

Step 18: Run Tools → Update PCB from Schematic (F8). Accept all changes. New footprints and modified footprints will appear.

Step 19: The old THT resistor and capacitor footprints will be replaced with their SMD equivalents. New switch connectors J5, J6 and their associated components will appear in the unplaced pile.

Phase F: Set Board Outline to 45×50mm

Step 20: Select all 4 Edge.Cuts lines → Delete them.

Step 21: Draw new board outline: select Edge.Cuts layer → draw rectangle. Start point: (131.5, 55), End point: (176.5, 105). This gives exactly 45mm × 50mm.

Step 22: Alternatively, use Place → Rectangle on Edge.Cuts layer with exact coordinates.

Phase G: Two-Sided Component Placement

Follow this placement strategy:

| Side | Layer | Components | Reason |
| --- | --- | --- | --- |
| TOP (F.Cu) | Front | HLK-2M05 (U1), Relays K1+K2, Connectors J1+J3+J4+J5+J6, MOV (RV1), Fuse (F1) | HV components + connectors (user-facing) |
| BOTTOM (B.Cu) | Back | ESP32-C6 (U2), AMS1117 (U6), HLW8012 (U5), PC817S (U3+U4), MMBT8050 (Q1+Q2), ALL SMD R/C, Test Points | LV logic + small SMD (reflow side) |

Step 23: Select each LV component → press F to flip to bottom layer. Do this for: U2, U3, U4, U5, U6, Q1, Q2, D1, D2, and all SMD resistors/capacitors.

Step 24: Place HV components on top: U1 (HLK-2M05) top-left, K1 and K2 side by side center-top, connectors along edges.

Step 25: Place switch connectors J5 and J6 on the right board edge, near the load connectors J3 and J4.

Phase H: Routing

Step 26: Route HV traces on F.Cu ONLY: mains input → fuse → HLK-2M05, relay contact paths to load connectors. Use 1mm trace width minimum for mains, 0.5mm for relay coil.

Step 27: Route LV traces on B.Cu ONLY: ESP32 signals, optocoupler connections, AMS1117 power, HLW8012 signals, switch inputs.

Step 28: Use vias where signals cross between layers: relay coil drive from Q1/Q2 (bottom) to K1/K2 (top). Via size: drill 0.3mm, pad 0.6mm for signals; drill 0.6mm, pad 1.2mm for power.

Step 29: Maintain 3mm minimum clearance between HV and LV zones on the top copper layer.

Phase I: Ground Pour & DRC

Step 30: Create GND copper pour on B.Cu: Add → Zone → select B.Cu → net = GND → draw rectangle covering the entire LV area. Edit → Fill All Zones (B).

Step 31: Run DRC: Inspect → Design Rules Checker. Fix all errors. Common issues: unconnected nets (check vias), clearance violations (adjust trace positions).

# 6. Connector & Terminal Layout (Aqara T1/T2 Style)

## 6.1 Terminal Assignment

Like the Aqara T1/T2, your module should have clearly labeled terminals:

| Terminal | Connector | Label | Wire | Function |
| --- | --- | --- | --- | --- |
| J1-Pin1 | 3-pin screw | L | Live (Brown) | 230V AC Live input |
| J1-Pin2 | 3-pin screw | N | Neutral (Blue) | 230V AC Neutral input |
| J1-Pin3 | 3-pin screw | L-OUT | Switched Live | Or use separate J for output |
| J3-Pin1 | 2-pin screw | L1_OUT | Load 1 Hot | Relay K1 switched output |
| J3-Pin2 | 2-pin screw | L1_COM | Load 1 Common | Relay K1 common (from L) |
| J4-Pin1 | 2-pin screw | L2_OUT | Load 2 Hot | Relay K2 switched output |
| J4-Pin2 | 2-pin screw | L2_COM | Load 2 Common | Relay K2 common (from L) |
| J5-Pin1 | 2-pin screw | S1 | Switch 1 signal | Wall Switch 1 input |
| J5-Pin2 | 2-pin screw | S1_GND | Switch 1 return | GND reference for SW1 |
| J6-Pin1 | 2-pin screw | S2 | Switch 2 signal | Wall Switch 2 input |
| J6-Pin2 | 2-pin screw | S2_GND | Switch 2 return | GND reference for SW2 |

⚠ WARNING: SAFETY CRITICAL: The wall switch terminals S1/S2 carry ONLY 3.3V DC logic signals, NOT mains voltage. The external wall switch connects between S and GND through the low-voltage circuit. This is fundamentally different from some budget smart relays that pass mains through the wall switch — our design is safer.

## 6.2 Wiring Diagrams (Text Description)

Method 1: Standard wiring with wall switch (like Aqara Diagram 1)

Mains L → J1-L → (through relay) → J3-L1_OUT → Load → J1-N

Wall Switch → connects between J5-S1 and J5-S1_GND

The module is placed inside the wall box behind the existing switch plate.

Method 2: No external switch (app/voice only)

Mains L → J1-L → (through relay) → J3-L1_OUT → Load → J1-N

J5 and J6 left unconnected — control via WiFi/app only.

# 7. Complete Bill of Materials (BOM)

| Ref | Value | Package | Description | LCSC/Source |
| --- | --- | --- | --- | --- |
| U1 | HLK-2M05 | Custom SMD | 5V 2W AC-DC converter module | HLK official / Aliexpress |
| U2 | XIAO-ESP32-C6 | DIP castellated | WiFi+BLE+Zigbee MCU module | Seeed Studio |
| U3, U4 | PC817S | SSOP-4 | Optocoupler (isolation) | C26154 (LCSC) |
| U5 | HLW8012 | SOIC-8 | Energy metering IC | C83906 (LCSC) |
| U6 | AMS1117-3.3 | SOT-223 | 3.3V LDO regulator | C6186 (LCSC) |
| K1, K2 | G5Q-1A DC5 | THT Relay | 5V 10A SPST-NO relay | Omron / Mouser |
| Q1, Q2 | MMBT8050 | SOT-23 | NPN transistor (relay driver) | C131862 (LCSC) |
| D1, D2 | 1N4007W | SMA | Flyback diode for relay | C64898 (LCSC) |
| F1 | 10A 250VAC | 1206 SMD | Fuse (mains protection) | C210398 (LCSC) |
| RV1 | 275VAC MOV | Disc 7mm THT | Varistor (surge protection) | C90154 (LCSC) |
| R1, R3 | 330Ω | 0603 | Optocoupler LED resistor | C23138 (LCSC) |
| R2, R5 | 10kΩ | 0402 | Pull-down resistor | C25744 (LCSC) |
| R4,R6-R8,R13 | 470kΩ | 0402 | HLW8012 voltage divider | C25790 (LCSC) |
| R9,R11,R12,R16,R17 | 1kΩ | 0402 | Various (bias, ESD series) | C11702 (LCSC) |
| R10 | 1mΩ 1% | 2512 | Current shunt (HLW8012) | C207154 (LCSC) |
| R14, R15 | 10kΩ | 0402 | Switch input pull-up | C25744 (LCSC) |
| C1,C3,C5-C8 | 100nF | 0402 | Bypass/decoupling capacitors | C1525 (LCSC) |
| C2, C4 | 10µF 10V | 0805 | Regulator bulk caps (X5R) | C15850 (LCSC) |
| J1 | 3-pin 3.5mm | THT screw term. | AC mains input | Degson DG250-3.5-03P |
| J3, J4 | 2-pin 3.5mm | THT screw term. | Load output | Degson DG250-3.5-02P |
| J5, J6 | 2-pin 3.5mm | THT screw term. | Wall switch input | Degson DG250-3.5-02P |
| TP1-TP4 | Test Point | THT 1.5mm pad | Programming/debug access | Standard test pad |

Estimated BOM cost (2-channel, per unit at 100pcs): ₹280-350 + ₹180-220 (XIAO ESP32-C6) = ₹460-570 total

PCB cost (JLCPCB, 45×50mm, 2-layer, 10pcs): ~₹150-200 including shipping

# 8. Safety & Compliance Checklist

Before sending to production, verify ALL of the following:

| # | Check | Status | Standard |
| --- | --- | --- | --- |
| 1 | Creepage distance L-N to LV ≥ 3mm on PCB | Must verify in DRC | IEC 60335-1 |
| 2 | Relay rated ≥ 10A @ 250VAC with UL/TÜV | ✓ G5Q-1A DC5 | UL 508 |
| 3 | Fuse rated 10A 250VAC (fast-blow) | ✓ F1 in 1206 | IEC 60127 |
| 4 | MOV surge protection on mains input | ✓ RV1 275VAC | IEC 61643 |
| 5 | Optocoupler isolation ≥ 3.75kV | ✓ PC817S (5kV) | IEC 60950 |
| 6 | HLK-2M05 isolation ≥ 3kV | ✓ (rated 3kVAC) | IEC 62368 |
| 7 | Flyback diodes on relay coils | ✓ D1, D2 across K1, K2 | Best practice |
| 8 | No LV traces on F.Cu in HV zone | Must verify in PCB | IEC 60335-1 |
| 9 | No HV traces on B.Cu | Must verify in PCB | IEC 60335-1 |
| 10 | Wall switch input is LV only (3.3V) | ✓ by design | Safety by design |
| 11 | Thermal relief on relay pads | Must verify in PCB | IPC-2221 |
| 12 | Silkscreen labels on all connectors | Must add in PCB | Best practice |

# 9. Manufacturing Output Checklist

## 9.1 Gerber Generation

Step 1: In PCB Editor: File → Fabrication Outputs → Gerbers

Step 2: Select layers: F.Cu, B.Cu, F.SilkS, B.SilkS, F.Mask, B.Mask, Edge.Cuts

Step 3: Click 'Generate Drill Files' → select Excellon format → Generate

Step 4: Zip all files → upload to JLCPCB/PCBWay

## 9.2 Assembly Files

Step 5: Generate BOM: Tools → Generate BOM → export CSV

Step 6: Generate Pick & Place: File → Fabrication Outputs → Component Placement

Step 7: Match component orientations to JLCPCB's rotation database

## 9.3 PCB Specifications for Ordering

| Parameter | Value |
| --- | --- |
| Board Size | 45mm × 50mm |
| Layers | 2 (double-sided) |
| Thickness | 1.6mm (standard) |
| Copper Weight | 1oz (35µm) — or 2oz for HV traces |
| Surface Finish | HASL (lead-free) or ENIG |
| Solder Mask | Green (standard) or Black (premium look) |
| Silkscreen | White, both sides |
| Min Track Width | 0.25mm (LV), 1.0mm (HV mains) |
| Min Clearance | 0.2mm (LV), 3.0mm (HV-LV boundary) |
| Min Via Drill | 0.3mm |
| Impedance Control | Not required |

# 10. Quick Reference — Complete Changes Summary

| Change # | What | From | To | Priority |
| --- | --- | --- | --- | --- |
| 1 | R1,R3 footprint | R_Axial_DIN0207 (THT) | R_0603_1608Metric (SMD) | HIGH |
| 2 | R2,R4-R9,R13 footprint | R_Axial_DIN0207 (THT) | R_0402_1005Metric (SMD) | HIGH |
| 3 | R10 footprint | R_Axial_DIN0207 (THT) | R_2512_6332Metric (SMD) | HIGH |
| 4 | C2,C4 footprint | C_Disc_D5.0mm (THT) | C_0805_2012Metric (SMD) | HIGH |
| 5 | C3,C5,C6 footprint | C_Disc_D5.0mm (THT) | C_0402_1005Metric (SMD) | HIGH |
| 6 | Add J5 (SW1_INPUT) | Not present | TerminalBlock 3.5mm 2-pin | HIGH |
| 7 | Add J6 (SW2_INPUT) | Not present | TerminalBlock 3.5mm 2-pin | HIGH |
| 8 | Add R14,R15 (pull-ups) | Not present | 10kΩ 0402 SMD | HIGH |
| 9 | Add R16,R17 (series) | Not present | 1kΩ 0402 SMD | MEDIUM |
| 10 | Add C7,C8 (filter) | Not present | 100nF 0402 SMD | MEDIUM |
| 11 | Board outline | 58.5×54.5mm | 45×50mm | HIGH |
| 12 | Flip LV to B.Cu | All on F.Cu | LV on B.Cu, HV on F.Cu | HIGH |
| 13 | Re-route all traces | Current routing | New 2-sided routing | HIGH |
| 14 | Add GND pour on B.Cu | Not present | Full B.Cu GND fill | MEDIUM |
| 15 | Add silkscreen labels | Minimal | All connectors labeled | LOW |

Total estimated time to complete all changes in KiCad: 4-6 hours for an experienced KiCad user, including routing.

— End of Document —