Smart Relay PCB

KiCad Component Migration Guide

60×60mm → 45×50mm  |  Indian Switchboard Fitment

Scope of This Guide 1. HLK-PM01 → HLK-2M05 (smaller PSU module) 2. HF3FF relay → Omron G5Q-1-DC5 (smaller relay) 3. THT passives → SMD (diodes, transistors, optocouplers) 4. 5.08mm connectors → 3.5mm pitch screw terminals 5. Two-sided PCB component placement strategy 6. Replace UART header J1 with test point pads

# Change 1  —  PSU: HLK-PM01 → HLK-2M05

Why this change? HLK-PM01 is 34×20mm. HLK-2M05 is 20×12mm — 65% smaller footprint, same 5V output.

## 1-A  Comparison

| Parameter | HLK-PM01 (Old) | HLK-2M05 (New) |
| --- | --- | --- |
| PCB footprint | 34 × 20 mm | 20 × 12 mm ✓ 65% smaller |
| Output | 5V / 600mA / 3W | 5V / 400mA / 2W |
| Input | 100–240VAC | 100–240VAC ✓ same |
| Pin pitch | 7.62mm (3 pairs) | 2.54mm castellated |
| Mounting | Through-hole DIP | SMD castellation / THT |
| Creepage | Built-in potting | 6mm min. on your PCB |
| Current enough? | 600mA — over-spec | 400mA — sufficient for ESP32-C6 + 2 relays (calc below) |

### Current Budget Verification  (HLK-2M05 at 400mA)

| Component | Typical mA | Peak mA |
| --- | --- | --- |
| ESP32-C6-MINI-1 (WiFi TX) | 80 | 240 |
| XIAO ESP32-C6 (active) | 150 | 300 |
| 2× Omron G5Q relay coils (5V, 180mW each) | 72 total | 72 |
| HLW8012 energy monitor | 8 | 10 |
| AMS1117 quiescent + 3.3V load | 20 | 30 |
| Total estimate | ~330mA | ~412mA ← just within 400mA |

⚠ Margin note: 400mA peak is tight. If both relays actuate simultaneously with ESP32 WiFi transmitting, you briefly hit ~412mA. The HLK-2M05 has a 10% overload tolerance so this is fine, but if you switch to XIAO module (not bare ESP32), add 100mA buffer and upgrade to HLK-1M05 (1W/200mA — same size) or solder a 220µF/16V capacitor on the 5V rail to handle transient peaks.

## 1-B  KiCad Schematic Changes  (Step by Step)

### Step 1 — Delete the old HLK-PM01 symbol

- Open schematic editor — double-click your .kicad_sch file in KiCad Project Manager

- Click the HLK-PM01 symbol (U1) to select it, then press Delete

- Also delete any PWR_FLAG symbols that were connected to its AC input side — you will re-add them to the new symbol

- Keep your net labels: MAINS_L, MAINS_N, +5V, GND — these are fine

### Step 2 — Find or create the HLK-2M05 symbol

- Press A (Add Symbol) to open the symbol picker

- Search for HLK-2M05 in the search box — if not found, proceed to Step 3

- If found: place the symbol, then skip to Step 4

### Step 3 — Create HLK-2M05 symbol manually (if not in library)

- Go to Tools → Symbol Editor (or press Ctrl+L)

- Click File → New Symbol → select your personal library (e.g. mylib.kicad_sym) → enter name HLK-2M05

- In Reference field enter U — in Value enter HLK-2M05

- Press P to add pins. Add these 6 pins:

| Pin # | Name | Type | Position | Connect to net |
| --- | --- | --- | --- | --- |
| 1 | L / AC_IN | Power Input | Left side, top | MAINS_L |
| 2 | N / AC_RET | Power Input | Left side, bottom | MAINS_N |
| 3 | +VO / +5V | Power Output | Right side, top | +5V |
| 4 | -VO / GND | Power Output | Right side, bottom | GND |
| (5) | FG / PE | Passive | Right side, middle | Earth/PE (if exposed) |

- Draw a rectangle around the pins: press R for rectangle, drag to cover all pins

- Press Ctrl+S to save the symbol to your library

- Close Symbol Editor, back in schematic press A, search HLK-2M05, place it

### Step 4 — Wire the new symbol

- Connect MAINS_L net label to the L/AC_IN pin

- Connect MAINS_N net label to the N/AC_RET pin

- Connect +5V PWR symbol to the +VO pin

- Connect GND PWR symbol to the -VO pin

- Add PWR_FLAG on both +5V and GND nets near this symbol (press A, search PWR_FLAG)

- Add a 100nF decoupling capacitor C_extra (0402 SMD) from +5V to GND near the output pins — this filters HLK switching noise

## 1-C  KiCad Footprint Assignment

- Double-click the HLK-2M05 symbol → click the footprint field → click the browse icon (...)

- In the footprint browser, search: HLK-2M05

- If not found, you will create it — proceed to next step

### Creating HLK-2M05 Footprint

- Go to Tools → Footprint Editor

- Click File → New Footprint → name it HLK-2M05 → choose SMD type

- Add the following pads using Add Pad (press O):

| Pad# | Name | Type | Position (mm) | Size (mm) |
| --- | --- | --- | --- | --- |
| 1 | L | THT Round | X=0, Y=0 | Drill 1.0, Pad 1.8 |
| 2 | N | THT Round | X=2.54, Y=0 | Drill 1.0, Pad 1.8 |
| 3 | +5V | THT Round | X=7.62, Y=0 | Drill 1.0, Pad 1.8 |
| 4 | GND | THT Round | X=10.16, Y=0 | Drill 1.0, Pad 1.8 |

⚠ Verify the exact pin spacing from the HLK-2M05 datasheet before committing to these coordinates. The above are approximate — Hi-Link's bare modules vary between batches. Download the official datasheet PDF and measure pin 1 to pin 4 spacing.

- Draw the module outline on F.Fab layer: a rectangle 20mm × 12mm centered on the pads

- Draw a courtyard on F.Courtyard: 21mm × 13mm (0.5mm clearance each side)

- Add keepout rule covering the module body: right-click → Add Rule Area → tick No copper pour, draw rectangle 20×12mm — this prevents copper fills under the module

- Press Ctrl+S, close Footprint Editor, reassign footprint to U1 in schematic

# Change 2  —  Relay: HF3FF → Omron G5Q-1-DC5

HF3FF = 19×15mm each. G5Q-1 = 12.5×7.5mm each. Same 10A/250VAC rating. Only footprint changes — schematic pin mapping is identical.

## 2-A  Pin Mapping Comparison

| Function | HF3FF Pin | G5Q-1 Pin | Notes |
| --- | --- | --- | --- |
| Coil + | 1 | 1 (A1) | Connect to relay driver transistor collector via D1 flyback |
| Coil − | 2 | 2 (A2) | Connect to GND |
| COM (Common) | 3 | 3 (COM) | Connect to MAINS_L or load common |
| NO (Normally Open) | 4 | 4 (NO) | Connect to switched output |
| NC (Normally Closed) | 5 | 5 (NC) | Optional — can leave no-connect |

✓ Schematic stays the same — pin numbers, names, and function are identical between HF3FF and G5Q-1. You only need to update the footprint, not re-draw any wires.

## 2-B  KiCad Schematic — Changing the Symbol

- Click on relay K1 symbol in your schematic

- Press E to open properties

- In the Value field: change HF3FF to G5Q-1-DC5

- Leave all pin connections as-is — no rewiring needed

- Repeat for K2

- Press Ctrl+S to save schematic

## 2-C  KiCad Footprint — Assign Omron G5Q-1 Footprint

- In schematic, double-click K1 → click the footprint field → click ...

- In footprint browser search bar type: G5Q

- If found: select Relay_THT:Relay_SPDT_Omron_G5Q → click OK

- If NOT found: you will create it — follow steps below

- Repeat for K2

### Creating Omron G5Q-1 Footprint Manually

- Open Footprint Editor:  Tools → Footprint Editor

- File → New Footprint → name: Relay_SPDT_Omron_G5Q-1  →  type: Through Hole

- Add 5 pads with the following layout (from G5Q-1 datasheet, imperial spacing):

| Pad# | Net Name | X (mm) | Y (mm) | Drill / Pad Size |
| --- | --- | --- | --- | --- |
| 1 | A1 (Coil+) | 0.00 | 0.00 | Drill 1.1mm, Pad 2.0mm round |
| 2 | A2 (Coil−) | 5.08 | 0.00 | Drill 1.1mm, Pad 2.0mm round |
| 3 | COM | 0.00 | −5.08 | Drill 1.3mm, Pad 2.2mm round |
| 4 | NO | 2.54 | −5.08 | Drill 1.3mm, Pad 2.2mm round |
| 5 | NC | 5.08 | −5.08 | Drill 1.3mm, Pad 2.2mm round |

Important: always verify these coordinates against the official Omron G5Q-1 datasheet (Omron document D20G5Q0603). Dimensions above are from standard G5Q footprint — the pin 1 dot mark on the physical relay must align with pad 1 in KiCad.

- Draw body outline on F.Fab: rectangle 12.5mm × 7.5mm, centered on pad array

- Draw courtyard on F.Courtyard: 13.5mm × 8.5mm (0.5mm all sides)

- Add pin 1 marker on F.Fab: small triangle or dot at pad 1 position

- Add silkscreen label on F.SilkS: text K1 centered on body

- Ctrl+S to save. Assign to K1 and K2 in schematic.

# Change 3  —  THT Passives → SMD

This is the highest-impact change for board area. Converting 6 THT components to SMD frees ~300mm² and eliminates large drill keepout zones.

## 3-A  SMD Replacement Summary

| Ref | Old (THT) | New (SMD) | Package | KiCad Footprint |
| --- | --- | --- | --- | --- |
| D1, D2 | 1N4007 DO-41 | 1N4007W or SS14 | SMA (DO-214AC) | Diode_SMD:D_SMA |
| Q1, Q2 | S8050 TO-92 | MMBT8050 | SOT-23 | Package_TO_SOT_SMD:SOT-23 |
| U4, U5* | PC817 DIP-4 | PC817S or EL817S | SMD-4 (SOP-4) | Package_SO:SSOP-4_4.4x3mm_P1.27mm |

* The optocoupler reference U4/U5 notation is from your project — adjust to match your actual reference designators (could be OK1, OK2, etc.).

## 3-B  Diodes D1, D2  —  1N4007 → 1N4007W SMA

### Step 1 — Update schematic symbol

- Click D1 → press E → change Value from 1N4007 to 1N4007W

- In the footprint field, clear the old THT footprint

- Click ... → search D_SMA → select Diode_SMD:D_SMA → OK

- Repeat for D2

Pin polarity unchanged: 1N4007W SMA has cathode (K) on the bar side and anode (A) on the other — same as DO-41. The band marker on the SMA package body matches the stripe on the DO-41. No schematic re-wiring needed.

### Step 2 — SMA footprint details

- Pad 1 (Anode): 2.2×2.7mm SMD

- Pad 2 (Cathode): 2.2×2.7mm SMD

- Pad spacing: 5.2mm center-to-center

- Courtyard: 7.6mm × 3.7mm

- The Diode_SMD:D_SMA footprint in KiCad's built-in library is correct — no custom footprint needed

## 3-C  Transistors Q1, Q2  —  S8050 → MMBT8050 SOT-23

### Step 1 — Update schematic symbol

- Click Q1 → press E → Value: change S8050 to MMBT8050

- Footprint field → click ... → search SOT-23 → select Package_TO_SOT_SMD:SOT-23

- Verify pin mapping is correct (critical — see table below)

- Repeat for Q2

### SOT-23 Pin Mapping — CRITICAL

| SOT-23 Pin | S8050 (TO-92) | MMBT8050 (SOT-23) | Notes |
| --- | --- | --- | --- |
| 1 | Emitter (E) | Base (B) | ⚠ DIFFERENT ORDER — check symbol |
| 2 | Base (B) | Emitter (E) | ⚠ DIFFERENT ORDER |
| 3 | Collector (C) | Collector (C) | ✓ Same |

⚠ Critical: S8050 (TO-92) pin order is E-B-C from left to right when flat face is toward you. MMBT8050 (SOT-23) pin order is B-E-C (pin 1=Base, 2=Emitter, 3=Collector). The KiCad SOT-23 symbol from the Device library already has the correct MMBT8050 mapping — but double-check that your existing Q1/Q2 symbol has B/E/C correctly wired. If you used a generic 'NPN' symbol, the pin numbers may need updating.

- In schematic: verify Base connects to optocoupler output, Collector connects to relay coil+, Emitter connects to GND

- If any pin assignment changed: delete the old wires and re-wire to the correct pins

## 3-D  Optocouplers  —  PC817 DIP-4 → PC817S  SOP-4

### Step 1 — Understand the package difference

- PC817 DIP-4: 4 pins, through-hole, 2.54mm pitch, body 5.6×3.6mm

- PC817S SOP-4: 4 pins, SMD, 1.27mm pitch, body 4.4×3.0mm

- Pin function: IDENTICAL — same 4 pins (Anode, Cathode, Emitter, Collector) in same order

### Step 2 — Update schematic symbol

- Click the PC817 symbol (e.g. U4) → press E

- Change Value from PC817 to PC817S

- In footprint field → click ... → search SSOP-4

- Select: Package_SO:SSOP-4_4.4x3mm_P1.27mm → click OK

- Alternative footprint search: SOP-4 or SOT-23-4

- Repeat for the second optocoupler symbol

### Step 3 — Verify pin connections

| Pin# | PC817 DIP | PC817S SOP-4 | Connection in your circuit |
| --- | --- | --- | --- |
| 1 | Anode (A) | Anode (A) | Via resistor from ESP32 GPIO |
| 2 | Cathode (K) | Cathode (K) | GND |
| 3 | Emitter (E) | Emitter (E) | GND (transistor side) |
| 4 | Collector (C) | Collector (C) | Via R to relay transistor base |

Note: PC817S SOP-4 package has the same pin numbering as PC817 DIP-4. No schematic rewiring needed — only footprint swap. The Package_SO:SSOP-4_4.4x3mm_P1.27mm KiCad built-in footprint is correct for PC817S.

# Change 4  —  Connectors: 5.08mm → 3.5mm Pitch

Standard Wago/Degson/Phoenix 3.5mm screw terminals are rated 10A/300V. Footprint is ~30% narrower than 5.08mm. Fully compatible with 1.5mm² (14AWG) wire for Indian mains wiring.

## 4-A  Connector Selection

| Connector | Old (5.08mm) | New (3.5mm) | KiCad Footprint |
| --- | --- | --- | --- |
| J4 Mains In (3-pin) | 5.08mm, 3P | 3.5mm, 3P | TerminalBlock_Degson:TerminalBlock_Degson_2EDGK-3.5-03P_1x03 |
| J5 Load Out (2-pin) | 5.08mm, 2P | 3.5mm, 2P | TerminalBlock_Degson:TerminalBlock_Degson_2EDGK-3.5-02P_1x02 |
| J2 Low-voltage | 5.08mm or 2.54mm | 2.54mm | No change — keep pin headers for LV side |

## 4-B  KiCad Steps — Updating Connector Footprints

- In schematic, double-click connector J4 → click the footprint field → ...

- In footprint browser, search: TerminalBlock_Degson or just 3.5

- For 3-pin mains connector, select: TerminalBlock_Degson:TerminalBlock_Degson_2EDGK-3.5-03P_1x03

- Alternatively search: TerminalBlock_Phoenix and pick a 3.5mm 3-pin variant

- Click OK → repeat for J5 (2-pin variant)

- Run ERC (E) to confirm no footprint errors

Indian market sourcing: Degson 2EDGK-3.5mm terminals are available from Robu.in, Evelta.com and Amazon India. Buy the plug+header set (male PCB header + female pluggable part) so the wires can be disconnected without desoldering. Search: Degson 2EDGK 3.5mm 3P

# Change 5  —  Two-Sided PCB Component Placement

Placing components on both sides of the PCB is the most effective way to halve the required board area without changing any schematic or values.

## 5-A  Which Components Go on Which Side  —  Placement Map

| Side | Components | Reason |
| --- | --- | --- |
| TOP (F.Cu) — HV Zone | J4 Mains In, J5 Load Out, F1 Fuse, RV1 MOV varistor, HLK-2M05 PSU, K1/K2 relays, R10 shunt, HLW8012 | HV components must be visible, accessible and inspectable. Relays click, so place on top. |
| BOTTOM (B.Cu) — LV Zone | XIAO / ESP32 module, AMS1117, Q1/Q2 transistors, D1/D2 diodes, PC817S optocouplers, all decoupling caps, all resistors | LV components are small SMD — perfect for bottom. Keeps LV zone physically separate from HV top side. |

## 5-B  Board Layout Diagram

TOP SIDE (Front / F.Cu) — 45mm wide ┌─────────────────────────────────────────────┐ │ ⚡HV ZONE (left 22mm) ║ 🔌RELAY ZONE │ │ ┌──────────┐ ┌────┐ ║ ┌────┐ ┌────┐ │ │ │ HLK-2M05│ │ F1 │ ║ │ K1 │ │ K2 │ │ │ │ 20×12mm│ │1206│ ║ │G5Q │ │G5Q │ │ │ └──────────┘ └────┘ [RV1]║ └────┘ └────┘ │ │ [J4 Mains In] [R10 shunt]║ [J5 Load Out] │ │ L N PE ═══3.5mm══════ ║ L1 L2 N PE │ │ [U5 HLW8012 top-right] │ └─────────────────────────────────────────────┘ BOTTOM SIDE (Back / B.Cu) — mirror view ┌─────────────────────────────────────────────┐ │ ✅ LV ZONE (all 3.3V/5V logic) │ │ ┌───────────────────┐ ┌───────┐ │ │ │ ESP32-C6 module │ │AMS1117│ │ │ │ 13.2 × 16.6 mm │ │SOT-223│ │ │ └───────────────────┘ └───────┘ │ │ [PC817S][PC817S] [Q1 Q2] [D1 D2] │ │ [R1~R9 0402/0603] [C1~C6 decoupling] │ │ [TP_TX TP_RX TP_GND ← UART test points] │ └─────────────────────────────────────────────┘

## 5-C  How to Flip Components to the Bottom in KiCad PCB Editor

- Open the PCB file: File → Open your .kicad_pcb file

- Run Tools → Update PCB from Schematic (F8) first to sync all new footprints

- To flip a component to the bottom side: click on it to select → press F (Flip). The component moves from F.Cu to B.Cu automatically

- For multiple components: click one, then Shift+Click the others → right-click → Flip

- After flipping, all F.Cu traces touching that component must be re-routed on B.Cu. KiCad will show ratsnest lines for the broken connections

### Layer Reference — What Each Layer Means

| KiCad Layer | Meaning | Your Use |
| --- | --- | --- |
| F.Cu | Front copper (top side) | HV components: relays, PSU, connectors |
| B.Cu | Back copper (bottom side) | LV components: MCU, transistors, resistors |
| F.SilkS | Front silkscreen | Ref designators, polarity marks, HV warning |
| B.SilkS | Back silkscreen | Ref designators for bottom components |
| F.Fab | Front fabrication layer | Component outlines (top) |
| B.Fab | Back fabrication layer | Component outlines (bottom) |
| In1.Cu, In2.Cu | Internal copper layers | Optional: GND pour, +5V pour (4-layer board) |

## 5-D  Design Rules for Two-Sided PCB

- HV copper on top, LV copper on bottom — never run an LV trace on the top copper layer in the HV zone, and never run an HV trace on the bottom side

- Vias = your friend: when a signal from the bottom (LV) needs to reach the top for connection (e.g., relay coil drive signal from transistor on bottom going to K1 on top), use a via. In KiCad: press V while routing to insert a via and switch layers

- Minimum via size for signal: drill 0.3mm, pad 0.6mm (your PCB manufacturer may require 0.2mm minimum drill)

- Minimum via size for power (5V, GND): drill 0.6mm, pad 1.2mm — use multiple vias (2–3) for high-current paths

- Copper pour: create a GND pour on B.Cu for the entire LV zone — this gives good return paths and EMC shielding. In KiCad: Add → Zone → select B.Cu → net = GND → draw rectangle

- Assembly note: if you plan to reflow solder at home, put SMD components on top first (reflowed), then hand-solder the bottom. If using a PCB assembly service, both sides can be reflowed with appropriate stencil design.

# Change 6  —  Replace UART Header J1 with Test Points

A 6-pin 2.54mm UART header J1 takes ~15×10mm. Replacing it with 6 test point pads takes ~6×4mm — a 4× space saving. You can still program and debug by probing the pads.

## 6-A  What Test Points Replace

| Old: J1 Pin | Signal | New: Test Pad Name | How to use |
| --- | --- | --- | --- |
| J1 Pin 1 | GND | TP_GND | Connect GND clip of USB-UART adapter |
| J1 Pin 2 | 3V3 | TP_3V3 | Optional power supply during programming |
| J1 Pin 3 | TX (ESP → PC) | TP_TX | Connect RX pin of USB-UART adapter |
| J1 Pin 4 | RX (PC → ESP) | TP_RX | Connect TX pin of USB-UART adapter |
| J1 Pin 5 | BOOT / GPIO9 | TP_BOOT | Pull LOW to enter bootloader mode — touch with probe to GND before power-on |
| J1 Pin 6 | EN / RESET | TP_EN | Pull LOW momentarily to reset ESP32 |

## 6-B  Step 1  —  Remove J1 Header from Schematic

- Open schematic → click the J1 header symbol → press Delete

- The wires connecting J1 to TX, RX, BOOT, EN, GND, 3V3 will now show as unconnected endpoints — this is expected

- Do NOT delete the net labels (TX, RX, etc.) on the ESP32 symbol side — you will reconnect these to test point symbols

## 6-C  Step 2  —  Add Test Point Symbols

- Press A to open symbol picker

- Search: TestPoint → select Device:TestPoint → place 6 copies

- Label each TestPoint symbol with the net name:

- TP1 → connect wire to GND net

- TP2 → connect wire to +3V3 net

- TP3 → connect wire to TX net (from ESP32)

- TP4 → connect wire to RX net (to ESP32)

- TP5 → connect wire to BOOT/GPIO9 net

- TP6 → connect wire to EN net

- Change the Value of each TestPoint symbol to its signal name: TP_GND, TP_3V3, TP_TX, etc.

- Change the Reference to: TP1, TP2, TP3, TP4, TP5, TP6

- Press Ctrl+S to save

## 6-D  Step 3  —  Assign Test Point Footprints

- Double-click TP1 → footprint field → ...

- Search: TestPoint in the footprint browser

- For production boards (solderable pad): select TestPoint:TestPoint_Pad_D1.5mm

- For pogo pin probing (with a barrel hole): select TestPoint:TestPoint_THTPad_D1.5mm_Drill0.7mm

- Assign the same footprint to all 6 test points

Recommended test point footprint comparison: • TestPoint_Pad_D1.5mm — flat SMD pad, 1.5mm diameter. Probe touches it. No hole. Use on bottom side. • TestPoint_Pad_D2.0mm — larger, easier to probe, still SMD. Good for GND pad. • TestPoint_THTPad_D1.5mm_Drill0.7mm — through-hole barrel. Pogo pin or hook clip fits in hole. Best for production test fixtures.

## 6-E  Step 4  —  Place Test Points in PCB Editor

- Run Update PCB from Schematic (F8)

- All 6 test point footprints appear in the unplaced pile — press T (ratsnest) to see their connections

- Place all test points on B.Cu (bottom side) in a row along one edge of the board

- Recommended placement: left or right short edge of the board, outside the main component area

- Group them together: TP_GND — TP_3V3 — TP_TX — TP_RX — TP_BOOT — TP_EN, in a 1.27mm or 1.5mm row

- Spacing: 1.27mm or 2.54mm pitch — consistent spacing allows a pogo-pin programming jig to be made

Test Point Layout on Bottom Edge (B.Cu) Bottom edge of PCB (viewed from back): ┌─────────────────────────────────────┐ │ (ESP32 and SMD components above) │ │ │ │ ○ ○ ○ ○ ○ ○ │ │ GND 3V3 TX RX BOOT EN │ │ ↑ ↑ ↑ ↑ ↑ ↑ │ │ 1.27mm pitch pads (SMD D=1.5mm) │ └─────────────────────────────────────┘ ← How to use: clip USB-UART GND clip to TP_GND, TX wire to TP_TX, RX wire to TP_RX, touch TP_BOOT to GND during power-on to enter flash mode, then upload firmware.

## 6-F  Silkscreen Labels for Test Points

- In PCB Editor, select each test point pad → right-click → Properties

- The reference (TP_GND, TP_TX etc.) should auto-appear on B.SilkS layer

- If text is too large, select it → press E → set size to 0.5mm / stroke 0.08mm

- Add a small arrow or line on B.SilkS pointing to pin 1 (TP_GND) so orientation is clear when connecting

# Final KiCad Workflow Checklist

Run through this checklist after completing all 6 component changes in KiCad.

## Schematic (KiCad Schematic Editor)

- HLK-PM01 replaced with HLK-2M05 symbol — 4 pins connected (L, N, +5V, GND)

- HF3FF relay symbols replaced with G5Q-1-DC5 (value field updated, footprint changed)

- 1N4007 DO-41 diodes footprint changed to Diode_SMD:D_SMA

- S8050 TO-92 transistors footprint changed to Package_TO_SOT_SMD:SOT-23 — pin B/E/C verified

- PC817 DIP-4 footprint changed to Package_SO:SSOP-4_4.4x3mm_P1.27mm

- Connector J4 footprint changed to 3.5mm pitch 3-pin terminal block

- Connector J5 footprint changed to 3.5mm pitch 2-pin terminal block

- J1 UART header deleted; 6 TestPoint symbols added with net connections

- ERC run — 0 errors expected (max 5 allowed warnings from previous session)

## PCB Layout (KiCad PCB Editor)

- Update PCB from Schematic (F8) — accept all changes

- New footprints placed in unallocated area (press Ctrl+T to see them)

- HLK-2M05 placed in HV zone top-left

- G5Q relays placed in HV zone top-right, side-by-side

- J4, J5 connectors placed on right board edge

- LV components (MCU, transistors, optocouplers, resistors, caps) flipped to B.Cu

- Test point pads TP1–TP6 placed in a row on bottom edge, B.Cu

- HV/LV 3mm creepage gap maintained on top side

- All traces re-routed (or updated if using auto-router)

- GND pour added to B.Cu  →  Edit → Fill All Zones (B)

- DRC run: Inspect → Design Rules Checker — 0 errors

- Board outline verified at 45×50mm maximum

## Manufacturing Output

- Gerbers: File → Fabrication Outputs → Gerbers — include F.Cu, B.Cu, F.SilkS, B.SilkS, F.Mask, B.Mask, Edge.Cuts

- Drill file: File → Fabrication Outputs → Drill Files

- BOM: Tools → Generate BOM — export CSV for parts ordering

- Pick & Place: File → Fabrication Outputs → Component Placement — for assembly house

Pro tip for Indian market: Order PCBs from JLCPCB or PCBWay. Both support 2-layer boards at this size for ~₹200–400 for 5 pieces with 7-day delivery. For assembly, use JLCPCB SMT service for bottom-side SMD components — they stock MMBT8050, 1N4007W SMA, and PC817S in their parts library. Upload your BOM and CPL (Component Placement) files to get an instant quote.